Docket No. 

Public Utility Commission of Texas

Direct Testimony of Stuart M. Nelson

In Support of

LCRA's Transmission Cost of Service Filing

Table of Contents

Qualifications 	1

Purpose	3

Transmission Planning	4

Adequacy of the LCRA’s Transmission System	7

Growth in Capital Plan	8

Functional Unbundling	9

The LCRA Capital Plan	14

Growth Operations and Maintenance Expenses	15

Transmission Lease	19

Substation Functionalization 	21

Allocation of SOCC and EMS Costs	22

Allocation of Dalchau Service Center Costs	23

Docket No. 

Public Utility Commission of Texas

Direct Testimony Of

Stuart M. Nelson

Lower Colorado River Authority

Q.	PLEASE STATE YOUR NAME AND BUSINESS ADDRESS?

A.	My name is Stuart M. Nelson.  My business address is Lower Colorado River Authority (LCRA), 3701 Lake Austin Boulevard, Austin, Texas 78703.

Q.	WHAT IS YOUR OCCUPATION?

A.	I am the Manager of Transmission Planning and Implementation for the LCRA.

Q.	IN YOUR PRESENT CAPACITY, WHAT ARE YOUR RESPONSIBILITIES?

A.	I am responsible for planning the additions to the transmission facilities owned by the LCRA and its wholesale customers.  I am also responsible for managing the LCRA’s transmission construction projects, along with the Surveying and Mapping Group.

Q.	WHAT IS YOUR EDUCATIONAL BACKGROUND?

A.	I graduated from the University of Texas at Austin with a Bachelor of Science Degree in Electrical Engineering in December 1983.

Q.	ARE YOU A REGISTERED PROFESSIONAL ENGINEER IN TEXAS?

A.	Yes, I received my certificate in October 1988 and my license number is 65139.

Q.	WHERE HAVE YOU BEEN EMPLOYED SINCE YOUR GRADUATION FROM COLLEGE?

A.	I worked at West Texas Utilities (WTU) as a Substation Design Engineer between February 1984 and May 1986.  I was employed by Central Power and Light (CPL) between May of 1986 and October of 1990 as a Transmission Planning Engineer, the Supervisor of Operational Studies and the Supervisor of System Engineering.  In October of 1990 I was employed by the LCRA as the Supervising Engineer of Transmission Planning.  In January, 1995, I was promoted to the position of Manager of Transmission Planning and Implementation.

PURPOSE

Q.	WHAT IS THE PURPOSE OF YOUR TESTIMONY?

A.	I will discuss the following subjects in my testimony:

- Transmission planning

- Adequacy of the LCRA’s transmission system

- Functional Unbundling

- Growth in the LCRA’s capital requirements for transmission projects

- The LCRA’s capital plan

- Growth in operations and maintenance expenses

- The LCRA’s lease of customer-owned transmission facilities

- Functionalization of substation costs

- Functionalization of costs associated with the LCRA’s Systems Operations Control Center (SOCC) and Energy Management System (EMS)

- Functionalization of costs associated with the LCRA ’s Dalchau Service Center

Q.	WHAT SCHEDULES ARE YOU SPONSORING?

A.	I am co-sponsoring Schedule F:  Functionalization factors (with Steve Bartley) for the Historic Test Year.

Q.	WAS THE SCHEDULE YOU ARE SPONSORING PREPARED BY YOU OR UNDER YOUR SUPERVISION?

A.	Yes, it was.

Q.	PLEASE DESCRIBE THE LCRA TRANSMISSION SYSTEM.

A:	The LCRA owns and operates 2,353 miles of transmission lines and 174 substations in 53 counties in Central Texas.  The LCRA also operates 892 miles of transmission lines and 90 substations through a lease agreement with our electric wholesale customers. The LCRA’s transmission system is situated between the major metropolitan areas of Austin, Houston and San Antonio and extends West to Kimble County, Texas.  The LCRA also owns a 26-mile long transmission line and one substation in Culberson County in far West Texas.  The LCRA transmission system is operated by TENSCO, which is an internal line of business within the LCRA.  When I refer to the LCRA or TENSCO, I am referring to the LCRA’s transmission service line of business, unless indicated otherwise by the context.

Q:	PLEASE DESCRIBE  TENSCO’S ORGANIZATIONAL STRUCTURE?

A:	An organizational chart for TENSCO can be found in Exhibit SMN-1. TENSCO is responsible for planning, operating, maintaining and building the LCRA’s transmission and transformation facilities.  TENSCO has approximately 400 employees at the LCRA General Office Complex and the Dalchau Service Center, both located in Austin, Texas. The five major work groups in TENSCO are Transmission Engineering, Field Operations, Transmission Planning and Implementation, System Operations, and Technical Support. TENSCO also has its own Regulatory, Human Resource, Safety, Environmental, Training, Business Management and Customer Relations staff.

TRANSMISSION PLANNING

Q.	DOES THE LCRA PERFORM TRANSMISSION PLANNING FOR ITSELF AND THE FACILITIES IT LEASES FROM ITS CUSTOMERS?

A.	Yes, the LCRA performs the transmission planning for itself and the facilities it leases through a joint planning process with its wholesale customers.  The LCRA also coordinates its planning process with the Electric Reliability Council of Texas (ERCOT) Independent System Operator (ISO).

Q.	DOES THE LCRA HAVE A TRANSMISSION PLAN?

A.	Yes, the LCRA’s Ten Year Transmission Plan is Documented in the Transmission System Improvements Plan (TSIP).

Q.	HOW ARE PROJECTS IN THE TSIP DEVELOPED?

A.	The projects in the TSIP are developed by the LCRA and its wholesale customers using the Transmission and Distribution Planning Criteria provided in Exhibit SMN-2.  Different groups are responsible for initiating certain types of projects.  The Transmission Planning and Implementation Group runs transmission load flow studies to find potential problems on the transmission systems owned by the LCRA and its wholesale customers.  Transmission Planning and Implementation reviews the wholesale customer work plans to see if the potential for joint projects exists.  Transmission Engineering provides rough order of magnitude cost estimates for the projects and alternatives that are being studied.  The Transmission Planning and Implementation Group consolidates the projects in the TSIP.

Q.	WHAT IS THE AWC POWER SUPPLY AND TRANSMISSION PLANNING COMMITTEE?

A.	The AWC Power Supply and Transmission Planning Committee is a group of representatives appointed by the LCRA’s Wholesale Customers.  The committee meets on a monthly basis and reviews the LCRA’s transmission construction, operations and maintenance practices.  The committee also reviews the justification for the transmission and substation projects, included in the TSIP, that are built or leased by the LCRA.

Q.	WHAT IS THE TRANSMISSION SYSTEM PLANNING TASK FORCE?

A.	The Transmission System Planning Task Force meets on an annual basis to review the TSIP and recommend projects to the LCRA Board of Directors for inclusion in the LCRA Business Plan.  Membership on the task force is open to representatives from all of the LCRA’s wholesale customers.

Q.	WHAT IS THE APPROVAL PROCESS FOR THE TSIP?

A.	The first rough draft of the TSIP is sent to selected staff and management at the LCRA for comment and review.  The comments are incorporated into a draft document that is reviewed by the Association of Wholesale Customers’ (AWC) Power Supply and Transmission Planning Committee.  After review by the AWC Power Supply and Transmission Planning Committee, the TSIP is reviewed by the Transmission System Planning Task Force and then presented to the LCRA Board of Directors.  The TSIP is a compilation of studies and recommendations made by the LCRA Transmission Planning and Implementation Department and does not provide authorization to spend money.  The ultimate authority to build or lease transmission facilities rests with the LCRA Board of Directors.

Q. 	ARE ALL TRANSMISSION PROJECTS BUILT OR LEASED BY THE LCRA TO SERVE ITS WHOLESALE CUSTOMERS REVIEWED IN THE TSIP?

A.	Yes, all transmission and substation projects built or leased by the LCRA to serve its wholesale customers must be reviewed and recommended in the Transmission System Improvements Plan.  The TSIP provides the Authorization and Justification Document (AJD) needed for a project to be carried to the implementation phase.  Once the AJD has been approved for a project being built by the LCRA, a project manager is assigned.  The project manager will compile construction estimates and carry the project to the LCRA Board for approval.

Q.	WILL THE LCRA LEASE TRANSMISSION OR SUBSTATION FACILITIES THAT HAVE NOT REVIEWED AND RECOMMENDED IN THE TSIP?

A.	No, the LCRA will only lease transmission and substation facilities that have been recommended in the TSIP.

Q.	HAVE ALL PROJECTS THAT HAVE BEEN ALLOCATED TO TRANSMISSION BEEN REVIEWED IN THE TSIP?

A.	No.  The TSIP did not include the Generation Interconnection, SOCC, EMS, Generation Metering, or Service Center Renovation Projects.

Q.	WHY ARE GENERATION INTERCONNECTION PROJECTS EXCLUDED FROM THE TSIP?

A.	The generation interconnection projects are being built to allow new generating plants to connect to the LCRA’s transmission system.  The LCRA has a statutory and regulatory obligation to provide open access to its transmission facilities.  The projects in the TSIP only cover transmission and substation facilities required for services to the LCRA’s wholesale customers and facilities in the Facilities and Premises Lease and Operating Agreement, referred to as the “Lease” later in my testimony.  Also, it would be very difficult to meet the confidentiality requirements in the ERCOT ISO’s Generation Interconnection Procedures with different groups within the LCRA and the wholesale customers reviewing the Generation Interconnection Projects in the TSIP.

Q.	WHAT PROCESS WAS FOLLOWED TO DEVELOP THE GENERATION INTERCONNECTION PROJECTS?

A.	The Generation Interconnection Projects are developed using ERCOT’s Generation Interconnection Procedures.

Q.	DID THE ERCOT ISO REVIEW THE LCRA’S GENERATION INTERCONNECTION PROJECTS?

A.	Yes, the ERCOT ISO has reviewed and approved the Security Studies performed by the LCRA to develop Generation Interconnection Projects described in the historic and forecast years.

Q.	WHY ARE THE SOCC, EMS AND SERVICE CENTER RENOVATION PROJECTS NOT INCLUDEDED IN THE TSIP?

A.	The TSIP reviews transmission line and substation projects that are needed to deliver power to the LCRA’s existing wholesale customers.

Q.	WHAT APPROVAL PROCESS DID THE SOCC, EMS AND SERVICE CENTER RENOVATION PROJECTS FOLLOW?

A.	The LCRA assembled work teams to evaluate the different alternatives for each project.  Outside consultants were retained to aid in the project evaluations.  The project recommendations were reviewed by the LCRA’s Financial Oversight Group and then placed in the LCRA’s Business Plan.  The LCRA Board of Directors reviews and approves all projects before significant expenditures are made.

ADEQUACY OF THE LCRA’S TRANSMISSION SYSTEM

Q.	ARE THE LCRA’S TRANSMISSION FACILITIES ADEQUATE TO PROVIDE RELIABLE TRANSMISSION SERVICE?

A.	Yes, the LCRA’s facilities are adequate when operated in coordination with other existing transmission systems in the ERCOT System.

Q.	ARE THE LCRA’S TRANSMISSION FACILITIES REASONABLY NECESSARY TO MEET THE CURRENT REQUIREMENTS OF THE INTERCONNECTED SYSTEM?

A.	Yes.  The LCRA’s transmission facilities are properly sized and configured to deliver energy to loads at substations connected to the LCRA’s transmission system and deliver energy from generators connected to the LCRA’s transmission system.  Due to its central location, the LCRA’s transmission system is also sized to transfer power from the major load centers of Houston, Dallas, Austin and San Antonio.

Q.	ARE THE LCRA’S PLANNED TRANSMISSION FACILITIES REASONABLY NECESSARY TO MEET THE SYSTEM NEEDS FOR THE 2002 FORECAST YEAR?

A.	Yes, the LCRA has planned its transmission system to meet future needs. The transmission facilities proposed in the LCRA’s capital plan are needed to meet the LCRA and ERCOT Planning Criteria along with the North American Electric Reliability Council’s (NERC) Planning Standards. The LCRA works with the ERCOT Independent System Operator (ISO) to determine the adequacy of the LCRA’s system and develops plans accordingly.

FUNCTIONAL UNBUNDLING

Q.  	IS THE LCRA SUBJECT TO FUNCTIONAL UNBUNDLING REQUIREMENTS RELATED TO ITS TRANSMISSION OPERATIONS?

A.  	Yes.  The LCRA operates an electric control area in the ERCOT, performing transmission and control area activities in compliance with NERC standards and ERCOT Operating Guides.  The LCRA is required to functionally separate the operation of its transmission facilities and the operation of its wholesale power purchase and sales activities.  The LCRA has filed with the Commission, and is in compliance with, its functional unbundling plan and "Standards of Conduct Concerning Non-Discriminatory Transmission Services," in accordance with P.U.C. Subst. R. 25.196.  A copy of the LCRA’s compliance filing is attached as Exhibit SMN-3.

GROWTH IN THE LCRA’s CAPITAL PLAN

Q.	WHAT ARE THE MAJOR ISSUES AFFECTING THE LCRA’S TRANSMISSION SYSTEM?

A.	The three major issues affecting the LCRA’s transmission system are: load growth, new generation interconnection projects and increasing transfers on the ERCOT system.

Q.	WHAT ARE THE GROWTH RATES OF LOADS AT SUBSTATIONS OPERATED BY THE LCRA?

A.	The loads at substations operated by the LCRA have experienced rapid growth since 1992, averaging 6.3 % per year.  The total LCRA system peak load has increased from 1,605 MW in 1992 to 2,454 MW in 1999.

Q.	HOW HAS THE LOAD GROWTH AFFECTED THE LCRA’S CAPITAL PLAN FOR PROJECTS THAT WILL BE COMPLETED BY DECEMBER 31, 2002?

A.	The LCRA has added an average of six new load-serving substations and transformer banks per year since 1992.  The LCRA and its wholesale customers expect to add at least 14 new substations and upgrade or add 20 power transformers to serve the LCRA’s wholesale customers by December 31, 2002.

Q.	HOW HAS THE INTERCONNECTION OF NEW GENERATORS AFFECTED THE LCRA CAPITAL PLAN FOR PROJECTS THAT WILL BE COMPLETED BY DECEMBER 31, 2002?

A.	The LCRA has signed interconnection agreements with four new power plants, and expects to sign at least two more, and all such projects will be “online” by the end of December, 2002.  The LCRA expects to have 4700 MW of new generation connected to its transmission system by December 31, 2002.  This represents a 161% increase over the 2920 MW of generation that is currently connected to the LCRA Transmission System.  The LCRA’s current capital plan includes a $32.6 million project to connect new wind generation facilities in West Texas to the ERCOT Transmission System.

Q.	HOW HAVE INCREASED TRANSACTIONS ON THE ERCOT SYSTEM AFFECTED THE LCRA’S TRANSMISSION PLAN FOR PROJECTS THAT WILL BE COMPLETED BY DECEMBER 31, 2002?

A.	New burdens on the ERCOT System from increased transactions have caused the LCRA to move up the addition of a second 345 kV circuit on the FPP to Lytton Springs 345kV line from June 2003 to June 2000.  The ERCOT ISO has reviewed and approved this project.  The LCRA will rebuild and upgrade the Zorn to McCarty Lane 138kV line and the McCarty Lane to San Marcos 138kV line  due to increased transactions in ERCOT.  The LCRA estimates that it will spend $13.9 million on these projects by December 31, 2002.

Q.	WHAT IS THE PURPOSE OF THE SOCC?

A.	The System Operations Control Center houses the LCRA’s SOCC Operations and Technical Support Groups and the Energy Management System. The main function of the SOCC is to monitor and control the operations of the LCRA’s transmission, transformation, generation and control area facilities.  The SOCC Operations Group is responsible for monitoring and controlling the LCRA’s facilities, responding to customer outages, and coordinating emergency operations with the ERCOT ISO.  The SOCC Operations Group has 29 positions, the majority of which are System Operators.  The LCRA has a minimum of three System Operators on duty 24 hours a day, 365 days a year.  When there are three Systems Operators on duty, there will be one Shift Supervisor, one Control Area Operator and one Transmission Operator.  A fourth System Operator is brought in to support transmission operations for twelve hours a day Monday through Thursday.  The Technical Support Group operates and maintains the LCRA’s Energy Management System.

Q.	WHY DID THE LCRA NEED TO BUILD A NEW SYSTEM OPERATIONS CONTROL CENTER?

A.	The old SOCC had been in service for 25 years, with around-the-clock usage.  During this period, several renovations were required due to operational changes, equipment replacement, equipment additions, and major facility repairs.  The old SOCC layout was not flexible enough to allow for needed modifications or expansion and had minimal security.  The facilities were no longer adequate for the existing and future operational requirements at the SOCC.  The transmission system was displayed on a physical map board in the control room that could no longer be expanded and was difficult to revise.  To remain functional, the old SOCC would have required extensive renovations that would have disrupted SOCC operations. The new SOCC meets the requirements of a changing industry and can be adapted to allow the LCRA to provide transmission and ancillary services required to serve the new open access electric market in ERCOT.

Q.	WHAT WAS THE APPROVAL PROCESS FOR THE SOCC PROJECT?

A.	The LCRA established a working group to evaluate the alternatives for upgrading the LCRA’s old SOCC and building a new SOCC.  The firm of Ball Resources, development and cost estimating consultants, was hired by the LCRA to develop cost estimates for the evaluation.  The LCRA’s Financial Oversight Group reviewed the recommendation to build the new SOCC.  The LCRA Board of Directors approved the first phase of the SOCC project in August 1998.  At that point, the firm of Ellerbe Becket was hired as the Architect to design the new SOCC.  The LCRA Board approved the construction of the new SOCC in March 1999.  The new SOCC went operational on May 8, 2000, and will provide much needed improvements in the security and reliability of the LCRA’s control center facility.

Q.	WHAT IS THE PURPOSE OF THE ENERGY MANAGEMENT SYSTEM?

A.	The EMS is a special purpose computer system used by the SOCC Operations staff to monitor and control the LCRA’s transmission, transformation, generation, and control area facilities.  The EMS communicates with 140 Remote Terminal Units (RTU’s) that monitor the status of equipment at substations operated by the LCRA.  The EMS allows SOCC’s transmission system operator to manage the LCRA’s transmission system by controlling devices at substations.  The EMS also economically dispatches the LCRA’s generators according to the direction of the LCRA’s generation merchant function, in compliance with functional unbundling requirements, and performs transmission control area functions.

Q.	WHY DID THE LCRA NEED TO INSTALL A NEW ENERGY MANAGEMENT SYSTEM?

A.	The old EMS was installed in 1985 and had reached the maximum capacity of data points that could be monitored.  If a new or more critical point needed to be monitored on the LCRA system, the LCRA had to stop monitoring an existing data point.  The supplier of the old EMS left the EMS business in 1991.  Support and parts became increasingly difficult to obtain, while failures increased due to the age of the equipment.  There was a significant risk that the old EMS could not be put back in service after the failure of a major component.  Additionally, the new EMS provides the tools needed by SOCC and other LCRA staff to respond to new and increasing demands in the deregulated electric utility environment.  The new EMS supports new ancillary services and EMS services evolving in the new deregulated environment, and it provides tools to facilitate improved system analysis.

Q.	WHAT WAS THE APPROVAL PROCESS FOR THE EMS PROJECT?

A.	An EMS Strategic Plan was developed in 1993, and a Project Team was formed in early 1994, with representatives from all concerned areas of the LCRA.  The team evaluated four alternatives: 1) maintaining the existing EMS, 2) gradually migrating to a new EMS, 3) building a new EMS with LCRA resources, and 4) buying a new EMS from an outside vendor.  After the analysis was completed, the team worked to develop preliminary requirements for the new EMS, and to perform a review of existing EMS technology.  This effort culminated in a contract with Energy Management Consulting Associates (EMCA) to pull together the team’s effort into an Internal LCRA Requirements Report and a Supplier Information Document, which was intended to be sent to qualified suppliers.  The LCRA Board of Directors approved the replacement of the Energy Management System (EMS) in December 1995.  The LCRA’s new Energy Management System (EMS) went operational in November 1999.

Q.	WHY DOES THE LCRA NEED TO REBUILD AND ADD NEW OFFICE SPACE AT THE LCRA’S DALCHAU SERVICE CENTER?

A.	The LCRA is faced with a shortage of office space and its transmission personnel are housed in two locations that are more than 11 miles apart.  The LCRA employed the architectural firm of Graeber, Simmons & Cowan to develop a master plan for how the LCRA should meets its facility requirements.  The evaluation reviewed alternatives, including expansion of the LCRA’s General Office Complex (GOC) and the Dalchau Service Center.  The architect worked with a team of LCRA staff made up of representatives from across the organization.  One of the main goals of the master plan was to address safety and traffic issues at the Dalchau Service Center.  The best plan to meet the LCRA’s office and facility needs is to build a new Fleet Maintenance Facility at the Dalchau Service Center and renovate the old shops to house personnel from the LCRA’s General Office Complex.  The Fleet Maintenance Facility will be built on a new piece of property adjacent to Ben White Boulevard (Highway 71).  The new location on Ben White Boulevard will provide the LCRA’s trucks and heavy equipment direct access to Highway 71, increasing traffic safety for the LCRA vehicles and the public.  When completed, the renovated buildings will provide office space for 200 additional employees at the Service Center.  Approximately 170 TENSCO and Telecommunications personnel will move from the LCRA’s GOC to the Service Center, resulting in efficiencies and room for growth at both sites.  An important benefit of this plan is that it consolidates all of the LCRA’s transmission personnel into one facility, while providing a natural separation between the LCRA’s transmission and generation businesses.

Q.	WHY IS THE LCRA INSTALLING BILLING QUALITY METERS AT ITS EXISTING POWER PLANTS?

A.	The new Standard Generation Interconnection Agreement, recently approved by the Commission, specifies that transmission providers will install billing quality meters at points of interconnection with new generators.  Also, the new settlement procedures being developed by ERCOT to facilitate a retail “choice” environment will necessitate generators having billing quality meters to record their output.

THE LCRA CAPITAL PLAN

Q.	WHAT ARE THE PROJECTED COSTS OF THE FACILITIES IN THE TSIP THAT WILL BE ADDED TO THE LCRA’S TRANSMISSION AND TRANSFORMATION EQUIPMENT BY DECEMBER 31, 2002?

A.	The cost estimates from the February 1999 and March 2000 TSIPs project that the LCRA will add $81,011,090 to its transmission and transformation facilities by December 31, 2002.

Q.	WHAT ARE THE PROJECTED COSTS OF THE LCRA’S GENERATION INTERCONNECTIONS PROJECTS THAT WILL BE IN SERVICE BY DECEMBER 31, 2002?

A.	The projected costs for the Generation Interconnection Projects are $53,082,100.

Q.	WHAT IS THE PROJECTED COST OF THE NEW SOCC?

A.	The new SOCC is projected to cost $14,900,000.

Q.	WHAT IS THE PROJECTED COST OF THE NEW EMS?

A.	The new EMS is projected to cost $6,390,000.

Q.	WHAT ARE THE PROJECTED COSTS OF THE RENOVATIONS AT THE DALCHAU SERVICE CENTER?

A.	The renovations at the Dalchau Service Center are projected to cost $28,319,000.  The LCRA will have spent $25,532,000 on renovations at the Service Center by December 31, 2002.

Q.	WHAT ARE THE PROJECTED COSTS TO INSTALL BILLING QUALITY METERS AT THE LCRA’S EXISTING GENERATION FACILITIES.

A.	The new billing quality meters at the LCRA’s existing plants are projected to cost $1,545,000.

GROWTH IN OPERATIONS AND MAINTENANCE EXPENSE

Q.	WHAT ARE THE MAJOR ISSUES AFFECTING TENSCO’S OPERATIONS AND MAINTENANCE EXPENSES?

A.	The major issues affecting TENSCO’s operations and maintenance expenses are the growth in the LCRA capital plan, loss of experienced personnel, difficulty in attracting qualified replacement personnel, and competitive wage adjustments.

Q.	HOW WILL THE LCRA’S CAPITAL PLAN AFFECT OPERATIONS AND MAINTENANCE EXPENSES?

A.	The LCRA’s capital plan has added new transmission facilities to the LCRA system that must be maintained.  Additionally, the LCRA maintenance personnel have been used to complete time-critical capital projects.  In response, TENSCO will increase its staff by approximately 10% between fiscal year 2000 and 2001 to meet this need.

Q.	WILL TENSCO TAKE ANY INTERIM STEPS TO ADDRESS ITS NEED FOR ADDITIONAL OPERATIONS AND MAINTENANCE PERSONNEL?

A.	TENSCO plans to employ outside contractors to assist with critical maintenance activities.  For example, TENSCO will use a tree-trimming contractor to help maintain its transmission line rights-of-way.

Q.	WILL TENSCO NEED TO USE OUTSIDE CONTRACTORS FOR CRITICAL MAINTENANCE ACTIVITIES BEYOND THE FORECAST TEST YEAR?

A.	Yes, TENSCO will need contract resources or additional staff beyond 2002 to accomplish critical maintenance activities, including transmission line right-of-way maintenance, which are needed to ensure transmission system safety and reliability.

Q.	HOW HAS THE LOSS OF EXPERIENCED PERSONNEL AND THE DIFFICULTY IN ATTRACTING NEW PERSONNEL AFFECTED TENSCO’S OPERATIONS AND MAINTENANCE?

A.	In calendar year 1999, TENSCO had a turnover rate of 14.4%, which was  double our 1997 turnover rate (7.2%).   Nearly 50% of the people who resigned or retired from key technical positions in 1999 cited compensation as a factor in their decision to leave.  The remaining 50% represented employees who were terminated, retired from non-key positions or were summer interns.  The biggest area of concern was the significant increase in turnover in TENSCO’s craft and technical ranks.  The turnover rate for TENSCO’s craft and technical ranks more than tripled, increasing from 3.7% (1997) to 9.3% (1998) to 12.2% (1999).  In 2000, TENSCO has already experienced a 6.4% turnover.  If this trend continues, TENSCO’s turnover rate will be 19.2 % by the end of the year.  Of the 24 employees who have resigned, retired or been terminated this year, eight or 33% of those cited compensation as a factor.  Also, TENSCO has averaged 10 – 20 posted job vacancies each month for the past three years.   Many of these were the same jobs from month to month.  On average, it has taken three months to fill vacancies, but for many technical positions, it has taken much longer.  For example, we have had civil and electrical engineering jobs posted for over six months and a system operations trainer posted for 18 months.  TENSCO must fill all of these vacant positions with qualified personnel to provide for a safe and reliable transmission system.

Q.	HOW DID TENSCO DETERMINE THAT IT NEEDED TO HAVE COMPETITIVE WAGE ADJUSTMENTS?

A.	As TENSCO attempted to recruit new employees to fill vacancies, it became apparent that the market for technical and engineering skills demanded higher wages.  Potential candidates also indicated that the cost of living had increased so much in the Austin area that relocation would be an issue.  The high turnover rate and the inability to fill open positions led TENSCO to perform a salary survey specifically for transmission positions.

### Q.	WHAT PROCESS DID TENSCO USE TO PERFORM ITS SALARY SURVEYS?

A.	TENSCO determined that it generally competes for technical employees with transmission utilities in Texas and the Southwest, including the ERCOT ISO.  For some positions, however, qualified applicants could only be found at the national level.  For example, after lengthy vacancies and extensive searches, we hired experienced system operators from Idaho and Colorado.  For certain other positions, such as administrative staff, entry-level craft positions, and computer programmers and technicians, TENSCO surveyed only the local Austin market, including non-utilities.  The most significant source of survey data was from the Southwest Personnel Group.  This data included benchmark jobs from 15 public and private electric utilities in the southwestern United States.  Ten of the participating organizations were from Texas.  Across all survey sources, TENSCO was able to appropriately match 50% of our job titles, covering all administrative employees, 70% of craft and technical employees and 80% of our professional employees.

### Q.	HAS THE LCRA PERFORMED SALARY SURVEYS IN THE PAST?

A.	Yes, TENSCO and the LCRA have performed general salary surveys every year, but placed a heavier weight on keeping salaries in line with workers in the LCRA service territory.

Q.	WHAT WERE THE RESULTS OF THE SALARY SURVEY?

A.	TENSCO found that its wage levels were 14.9% and 16.5%, respectively, below the survey average for craft and technical positions, and professional and managerial positions.  The System Operators at the SOCC were 42.3% below the average for system operators in the survey.  In Field Operations Line Electricians were 25.5% below average in the survey and District Operators were 20.3% below average in the survey.  The survey also showed that TENSCO’s Engineering Associates II, Engineers and Senior Engineers were 11.5%, 13.5% and 13.9% below average in the survey respectively.  A summary of the survey results is found in Exhibit SMN-4.

Q.	WHAT DID TENSCO DO AS A RESULT OF THE SALARY SURVEY?

A.	TENSCO adjusted existing employee salaries, starting in February 1999, in order to keep our current employees and to assure equity when bringing in newer, less experienced personnel.  The LCRA is raising its pay scale, including TENSCO, based upon receiving the results of this and other surveys.

Q.	ARE THE LCRA’S TRANSMISSION OPERATIONS AND MAINTENANCE EXPENSE REASONABLE AND NECESSARY?

A.	Yes.  Additionally, the LCRA’s operations and maintenance expenses are comparable to the operations and maintenance expenses contained in other transmission providers’ UCOS filings.  The LCRA has compared its operations and maintenance expense per $100 of Net Transmission Plant in Service with the two largest transmission providers for the historic and forecast year:

Historic		Forecast

LCRA		$6.34			$5.55

TXU		$7.20			$5.90

Reliant	$4.83			$5.79

TRANSMISSION LEASE

Q.	WHAT IS THE TRANSMISSION LEASE?

A.	The LCRA leases transmission and transformation facilities from some of its wholesale customers pursuant to a standard written lease.  The Transmission Lease, more accurately called the Facilities and Premises Lease and Operating Agreement, is self-explanatory.  Section 2.1 of the Lease grants the LCRA "the exclusive right to use, connect to and operate Lessor's Facilities…together with the premises upon which such Facilities are situated, whether by easement of by fee, as part of the LCRA's transmission system."  For this right, the LCRA pays a monthly fee, based on the book value of the leased facilities.  The Lease was approved by the Commission in Docket No. 9427, and the Lease enables the LCRA, with its customers, to plan and operate facilities owned by several entities as a single system.

Q.	WHAT ARE SOME OF THE BENEFITS OF THE LEASE AGREEMENT?

A.	The lease established a joint planning process that has saved the LCRA and its wholesale customers over $82 million by reducing the number of capital projects proposed in ten year plans developed before 1991.  The joint planning process also allowed the LCRA and its wholesale customers to postpone $58 million worth of capital projects proposed in capital plans developed before 1991.  The lease also allowed the LCRA to monitor and operate the transmission systems owned by the LCRA’s wholesale customers.

Q.	HAS THE LEASE BEEN REVIEWED BY THE COMMISSION IN PREVIOUS COMMISSION PROCEEDINGS?

A.	Yes, the lease was reviewed in Commission Dockets Nos. 9427 and 15640.  In Docket No. 9427, the Commission found that the Lease arrangements with LCRA customers for providing transmission service was in the public interest and the method for calculating payments under the Lease was reasonable.

Q.	HOW MUCH OF THE EQUIPMENT THAT THE LCRA LEASES IS TRANSMISSION EQUIPMENT AS DEFINED BY P.U.C. SUBST. R. 25.19?

A.	Approximately 65.1% of the equipment that is included in the lease meets the definition of transmission facilities under P.U.C. Subst. R. 25.19.  The non-transmission amounts represent the costs associated with transformation facilities that the LCRA leases from its customers that would be classified as wholesale distribution facilities.  The cost of such non-transmission facilities is included in the LCRA's Wholesale Transformation Service tariff.

Q.	BY WHAT PROCESS IS THE COST OF TRANSMISSION EQUIPMENT SEPARATED FROM NON-TRANSMISSION EQUIPMENT IN THE LEASE?

A.	In setting up the transmission lease calculation, three categories were established: transmission, transformation, and distribution.  All distribution equipment, primarily oil circuit breakers and oil circuit reclosers, is considered non-leasable, whereas transmission and transformation items are considered leasable.

Q.	HOW WERE COSTS ALLOCATED BETWEEN TRANSMISSION AND DISTRIBUTION FOR NEW SUBSTATIONS IN THE LEASE?

A.	A similar process was used to allocate the Transmission Lease to the transmission function as was used to allocate the LCRA's capital costs for substations.

SUBSTATION FUNCTIONALIZATION

Q.	PLEASE EXPLAIN THE PROCESS USED TO FUNCTIONALIZE SUBSTATION EQUIPMENT TO TRANSMISSION, WHOLESALE DISTRIBUTION, AND RETAIL DISTRIBUTION.

A.	A one-line diagram was prepared by the LCRA's Transmission Engineering Department for each of the LCRA's substations and those substations leased by the LCRA.  These diagrams split each substation into transmission facilities (based upon voltages of 60 kV or above, per P.U.C. Subst. R. 25.19), wholesale distribution facilities (those facilities below 60 kV that are used to serve the LCRA' s customers at wholesale), and retail (that small portion of facilities used to serve the LCRA's three retail customers).

Q.	HOW WERE THE ONE-LINE DIAGRAMS USED TO ASSIGN SUBSTATION COSTS TO THE DIFFERENT FUNCTIONS?

A.	The LCRA’s station equipment booked in certain FERC transmission accounts was allocated to each substation, based upon standard module costs.  The total station equipment booked in these FERC accounts for each customer was multiplied by that customer’s composite substation allocation factor.  These module costs were split by function (transmission, distribution, and retail) and the booked costs for each function for each substation were allocated based upon the percentage of estimated module costs in each function to the total estimated module costs for each substation.

Q.	IS THIS METHOD FOR ALLOCATING SUBSTATION COSTS REASONABLE?

A.	Yes, the same method was used to allocate substation costs in the LCRA’s last transmission cost of service filing in Commission Docket No. 15640.  The module costs are the same cost estimates that the LCRA uses for transmission project estimates for itself and its customers and the module components are in basic facility units.  The module costs are relative numbers also.  That is, the costs are used to allocate the actual original cost of the station equipment on the books.  This method of allocating substation costs has been used by the LCRA in past rate cases, and has not been a controversial subject in those cases.

Q.	WHAT COSTS WERE CONSIDERED TO BE COMMON COSTS AND HOW WERE THOSE COSTS FUNCTIONALIZED?

A.	The cost of the land, booked in FERC Account 350, the cost of structures and improvements, such as the site itself and the control house, booked in FERC Account 352, and certain costs, such as grounding and engineering, booked in FERC Account 353, were considered to be common costs.  Common costs were allocated to transmission based on the ratio of the replacement cost of substation facilities assigned to the transmission function to the total replacement cost of equipment assigned to the transmission, wholesale distribution, and retail functions.  This method of allocating common costs is the method specified in the Commission's Transmission Cost of Service Filing Package instruction eleven.

ALLOCATION OF SOCC AND EMS COSTS

Q.	WHAT PORTION OF THE COSTS FOR BUILDING AND OPERATING THE LCRA’S SOCC AND EMS ARE ALLOCATED TO TRANSMISSION?

A.	Seventy-six and three tenths of a percent (76.3%) of the construction and operating costs of the SOCC and EMS have been allocated to transmission.

Q. 	HOW WAS THIS ALLOCATION DERIVED?

A.	The time spent by the SOCC Operations and Technical Support groups on different activities was tracked. The work performed by these two groups was allocated to generation and control area activities.  The allocation factor was derived from the weighted average cost of the time spent by these two groups.  The personnel at the SOCC currently spend 85.6% of their time on control area activities. The control area portion of the SOCC costs were then allocated between transmission (89%) and transformation (11%) based upon the LCRA’s value in these asset classes (see the testimony of Steve Bartley).

Q.	IS THIS METHOD FOR ALLOCATING THE SOCC AND EMS COSTS REASONABLE?

A.	Yes, this is the same method the LCRA used to develop its Generation Host Control Area Services Tariff (Control No. 21916) and Dynamic Scheduling Tariff (Control No. 21040).

Q.	IS THE SAME ALLOCATION FACTOR USED FOR THE HISTORIC AND FUTURE TEST YEARS?

A.	Yes.

ALLOCATION OF DALCHAU SERVICE CENTER COSTS

Q.	WHAT PORTION OF THE COSTS OF THE DALCHAU SERVICE CENTER IS ALLOCATED TO TRANSMISSION?

A.	Eighty-one and two tenths percent (81.2%) of the costs of the Dalchau Service Center are allocated to transmission for the historic test year.  Seventy-six and one tenth percent (76.1%) of the costs of the renovated Dalchau Service Center are allocated to transmission in the projected test year.

Q.	HOW WERE THESE ALLOCATIONS DERIVED?

A.	The surface area (square feet) of the buildings at the service center was allocated to the lines of business based upon the use of the buildings.  The surface areas (square feet) of the SOCC and Telecommunications Operations Center (TOC) were then allocated to the lines of business based upon the specific factors developed for the SOCC (85.6%) and telecommunications facilities (53.5%).  Common facilities at the service center that could not be directly assigned to a line of business were allocated based upon the relative asset values in direct plant (22.8% for TENSCO).  TENSCO’s portion of the Service Center costs were then allocated between transmission (89%) and transformation (11%) based upon the LCRA’s value in these asset classes (see the testimony of Steve Bartley).

Q.	DOES THIS CONCLUDE YOUR TESTIMONY?

A	Yes.

AFFIDAVIT OF STUART M. NELSON

§

STATE OF TEXAS		§

§

BEFORE ME, the undersigned authority, Stuart Nelson, being first duly sworn, deposes and states:

"My name is Stuart M. Nelson. I am employed as Manager of Transmission Planning and Implementation for the Lower Colorado River Authority, having its principal place of business at 3701 Lake Austin Boulevard, Austin, Texas.  I am over the age of twenty-one and am competent to make the following affidavit:

The foregoing testimony offered by me is true and correct and the opinions stated therein are, in my judgment and based upon my professional experience true and correct."

Stuart M. Nelson

SUBSCRIBED AND SWORN TO BEFORE ME this             day of May, 2000.

Notary Public, State of Texas