HARD TO REACH CUSTOMERS

ENERGY EFFICIENCY PROGRAM

Eligible Measures and Installation Criteria

| CONTROL ENVELOPE ENERGY WASTE |  |  |
| --- | --- | --- |
| MEASURE | MATERIAL | MINIMUM CRITERIA FOR INSTALLATION |
| Infiltration | Acrylic Latex plus Silicone Sealing Compounds Caulk Polyurethane Foam Elastomeric Sealant (including polysulfide, polyurethane and silicone) caulk | - Conforms to ASTM C834-95 with silicone - Conforms to ASTM C920-98 - Conforms to ASTM C920-98 - All visible caulk should be clear - A minimum 10 year life expectancy for ALL materials used. |
| Weather-stripping | Foam Tape Rigid Gaskets | - Must be closed cell - Must be UV resistant - Must be adjustable and attached permanently. - Gaskets must be attached to an aluminum carrier. - A minimum 10 year life expectancy for ALL materials used. |
| Insulation | Mineral Fiber Blanket Mineral Fiber Loose Fill Cellulose Vermiculite (loose fill) Perlite (loose fill) R-Value | - Conforms to ASTM C665-98 - Conforms to ASTM C764-98 - Conforms to ASTM C739-97 - Conforms to ASTM C1149-97 - Conforms to ASTM C516-80 - Conforms to ASTM C549-81 - Existing level of R-22 is in compliance with program. - Added insulation above R-22 must be installed to R-30. |
| MEASURE | MATERIAL | MINIMUM CRITERIA FOR INSTALLATION |
| ## Insulating R-values | Walls Knee wall Floor (Pier & Beam) | - Install to R-13 - Install to R-19 - Install to R-15 |
| ## Duct Sealing | Fiber Backed Mastic Compound Replacement Duct | - Apply to the duct joint - Replace with R-4 insulated duct. - A minimum 10 year life expectancy for ALL materials used. |
| CONTROL INTERIOR ENERGY USAGE |  |  |
| MEASURE | MATERIAL | MINIMUM CRITERIA FOR INSTALLATION |
| Lighting | Compact Fluorescent Lighting | - All CFLs to have a 10,000 hour lifetime burn. - 82 CRI (color rendition index). - 2700 Kelvin (color temperature) - A minimum 7 year life expectancy for all CFLs |
| Water Savers | Kitchen Aerators Bath Aerators Showerhead | - 1.5 gallons per minute maximum - 1.5 gallons per minute maximum - 2.5 gallons per minute maximum - Self cleaning |
| Refrigerators | Two Door/High Efficiency | - 14,16 & 18 cubic feet offered - Maximum size 18 cubic feet - 1,000 kWh annual difference between existing and replacement unit |
| CONTROL HVAC EFFICIENCY |  |  |
| MEASURE | MATERIAL | MINIMUM CRITERIA FOR INSTALLATION |
| Heating, Ventilation and Air Conditioning (HVAC) | Window Unit Packaged Units Split System | - 10 EER minimum - 12 SEER minimum - 12 SEER minimum |
| DIAGNOSTIC MEASURE | MATERIAL | MINIMUM CRITERIA |
| Health & Safety | CO Testing Device | - Test every housing unit that has combustion equipment. - Ambient air CO levels must not exceed 9 parts per million at project completion. |
| Air Infiltration/Indoor Air Quality | Blower Door Testing | - 30% air infiltration reduction at project completion. - The final air exchange for an individual unit must not be below 1,500 cubic feet per minute. - Measurements pressure differential of 50 pascals. |