Framework for a Uniform Rate Design

| ## Type of Charge | Metering | Measurement | Units | Demand Interval |
| --- | --- | --- | --- | --- |
|  Metering |  |  Monthly |  Per ESI id |  Per ESI id |
|  Billing |  |  |  |  |
|  Other Charge |  |  |  |  |
|  Transmission |  IDR | - 4CP ERCOT - 12CP Non-ERCOT |  kW |  60 min |
|  |  Indicating Demand |  NCP Any Time |  kW  kVA |  15 min |
|  |  kWh Only |  Energy |  kWh |  N/A |
|  Distribution |  IDR |  NCP On Peak |  kW  kVA |  15 min |
|  |  Indicating Demand |  NCP Any Time |  |  |
|  |  kWh Only |  Energy |  kWh |  N/A |
|  Minimum |  Any |  Fixed Amount |  Metering and Billing |  |
|  |  |  Demand Ratchet |  85% NCP  100% rolling avg. 4CP ERCOT (12CP Non-ERCOT) |  |
|  Power Factor Penalty |  Applicable to kW-based rates |  |  Up to 95% |  |
|  TCRF |  Annual based on changes in the wholesale postage stamp rate. |  |  Same as Transmission Charge |  |