SOAH DOCKET NO. 473-17-2686

PUC DOCKET NO. 46831

APPLICATION OF EL PASO	§	BEFORE THE STATE OFFICE

ELECTRIC COMPANY TO	§	OF

CHANGE RATES		§	ADMINISTRATIVE HEARINGS

EL PASO ELECTRIC COMPANY'S RESPONSE TO

FREEPORT MCMORAN INC.'S SECOND REQUEST FOR INFORMATION

QUESTION NOS. FMI 2-1 THROUGH FMI 2-20

FMI 2-1:

Explain fully the difference between the system load factor of 50.67% in Schedule O-01.06 and the system load factor of 51.9255% on page 4 of 12 in Workpaper P-07.

RESPONSE:

The difference between the system load factor of 50.67% shown in Schedule O-01.06 and the system load factor of 51.9255% reported on page 4 in Workpaper P-7 is due to the following reasons:

- The native system MWh represents the actual energy produced at source for the test year, while energy used in calculating the system load factor shown in Workpaper P-7 is the adjusted “at source” energy from Schedule O-1.4 (pages  21-24). The adjusted “at source” energy has been annualized for customers and has been adjusted for weather and energy efficiency. See page 4 line 8 through page 13 line 9 of the direct testimony of EPE witness Manuel Carrasco for a description of the customer annualization process and the weather and energy efficiency adjustments.

- The native system MW of 1,892 shown in Schedule O-1.6 is also different from the demand used in calculating the system load factor of 51.9255% which is the adjusted coincident demand “at source” from Schedule O-1.4 (pages 5-8). EPE uses the estimated class load and coincidence factors with the adjusted energy that has been annualized for customers and adjusted for energy efficiency and weather normalized to estimate monthly maximum and coincident demand by rate class and jurisdiction.

- Energy and demand used in calculating the load factor shown in Workpaper P-7 does not include interruptible energy and demand.

Preparer:	Enedina Soto	Title: 	Economist - Senior

Sponsor:	George Novela	Title: 	Manager - Economic Research