Corix Interviews 1-31-19 and 2-5-19

Darrin, Mary, Dennis Daniel, Scott, Matt, Tommy,

3 systems that account for most of the assets – Mitchel County (around 270 miles), Lometa (around 270 miles), Northeast Washington Co. - 800 miles in total with 3000 connections. All have similar characteristics build in 1970s or 1980s.  Most miles are small diameter pipe (2” to 6”) and primarily PVC and with less than optimal construction practices.

Structures
Source of supply –DW - life is reasonable at 30 years – most are on rivers and would not last through multiple floods and currents will impact.  They are primarily steel and some are moveable intakes.  There are some assets in the account that were incorrectly mapped to that account and will be moved.
T&D - 
Water Treatment – Most of the investment is new (metal buildings, piping, foundations, basins, etc.) – Ridge Harbor and Lake Buchanan – they would expect 50 years for these assets.
T&D – most $ should be moved to another account – DW - 50 years is reasonable for the remaining investment
General Plant -   DW - 50 years is also appropriate for this group – similar assets in this account to WT account.

Wells
Pumps are replaced 2-5 years.  Most of the value is in the drilling of the wells – which have a long life.  The current 7 year life is too short.  40 years is a more reasonable average which balances the equipment lives (some as low as 2 years) with the well itself (which may last 50 years or more). There is some risk of floods shortening the life of some of the assets as well.

Supply Mains
There is no functional difference between this and water mains.  DW - Use the 60 year life set for mains.

Pumps
Source/Treatment/T&D – 25 years is reasonable for all three accounts – there is no material difference between the characteristics of the assets in the three groups.  Impeller failure is a large cause of pump failure and winding failure is a large reason for motor failure.  They would be replaced independently.  Impellers and windings may last anywhere from 15 to 30 years and retirements would happen on being rebuilt.

Water Treatment Equipment
Clarifiers, rakes, arms, chemical systems, filters, tri-mite treatment units.  The current life of 20 is too short.  Some steel tanks may last 40 years. The treatment units will need substantial overhauls in the 20-30 year time frame.  Clarifiers (motors may have a 10 year life but rakes/arms can last 30 years or more). Dw - A 30 year average life is appropriate.  Use a fairly wide dispersion.

Reservoirs
Storage tanks have a relatively long life.  They would expect a 50-60 year life for the tanks.  There is some other equipment in the account that would have a shorter life (such as hydro tanks with a 20 year life).  DW - A 60 year life average is appropriate for this account.

Mains  
There are some systems with a shorter expected life than others.  The Northeast Washington and systems has problems but is already 40-45 years old. DW -  Move to 60 years – average for smaller pipe is 68 years but the systems are not always robust.

Service Lines
There is a different construction standard, not as thick walled and buried less deep than mains (which increases the risk of dig-ins or other disruptions).  They would expect a shorter life than mains.  DW - A 10-year differential is reasonable so use 50 years for this account.

Meters/Installation
Meters are AMR enabled (2/3 of meters are AMR).  They can replace the communication module independently.  Meters can last 20 years or more.  Batteries are expected to last 20 years.  They will be replacing 1/3 of their meters over the next couple years – those meters are at least 20 years old.  A 20 year life for meters is appropriate.  
DW - Set meter installation to the same life as meters – 20 years.

Hydrants
They do not provide fire protection services.  These are for flushing the system.  DW - Continuing to use the existing 35 year life is appropriate.

Backflow preventer
They would expect a life of 20-25 years for these valves.  DW - Set the life to 25 years.

Other
These accounts have the same type of equipment as Water Plant treatment.  DW – use life of 30 years.

Waste Water

Structures
Treatment – The environment is much more corrosive than water.  A 10-year shorter life than water structures.  DW - 40 year life is reasonable.

Power Gen Equip
Genset Diesel generator – they would expect a 30 year life for this asset.

Mains  
Force/Gravity – They would expect force mains to have a similar life as gravity (due to gravity mains having manholes - with a shorter life) and a similar life than water mains.  DW - Set both to 60 years.

Manholes
Most of the investment is in a pig launcher.  They generally will rehab a manhole every 20 years (rebuilding and recoating).  Use the same life as mains - 60 years.  This account will be cleared out and doesn’t need to have a life/rate set.

Special Collection Structures
All should be in the Treatment Equipment account.

Services to Customers
These are sewer taps – use the same life as mains.  This is different from water services in that there is no meter, the pipe is thicker and it is buried deeper.  DW - Use 60 years.

Flow Measuring Devices
Flow meters are generally outside of the effluent flow and could last up to 20 years.  DW - Use 20 years.

Treatment/Disposal Equipment
Lagoon – Only 24K or so in assets that will remain in this account (irrigation pond intake and Aeration equipment).  The remaining equipment should last 20-25 years.  Use 25 years
WW Plant – Membranes included in this account have only lasted 15 years or less.  Newer technology will last longer.  The UV lamps would only have a 10 year life.  DW - The same basic lives for the water equipment would be expected for these assets.  However, the environment is more corrosive than water so using a 25 year life as compared to a 30 year life for water is appropriate.  Camp Swift is a substantial portion of the assets and is irrigation (with a shorter life).

Receiving Wells
The current life of 20 years is too low.  The lift stations that are being rebuild are around 40 years old.  The corrosive environment will eventually fail and the bottom will deteriorate.  DW - A 40 year life is reasonable.

Pumping Equipment
25 years is reasonable for all three accounts – there is no material difference between the characteristics of the assets in the three groups.  Impeller failure is a large cause of pump failure (more likely in WW but the impellers are designed for that type of service) and winding failure is a large reason for motor failure.  They would be replaced independently.  Impellers and windings may last anywhere from 15 to 30 years and retirements would happen on being rebuilt.

Plant Sewer Equip
DW - Make the life the same as 1400 (Treatment Equipment)

Other Collection/Treatment
DW - Make the life the same as 1400 (Treatment equipment)

General Plant

Office Structures

1175 – DW - 40 years to match 1612 – same assets

Office Equipment

1621 is Mitchell County and has computers as well as other equipment.  A 7 year life is reasonable for the mix of computers and longer life assets. 
1180 and 1460 includes more furniture . DW – a 15 year life for both is reasonable.

Computer Assets

1585 is computer hardware (except for Mitchell Co.) and 1590 is software.  1585 is currently 3 years – which is too short.  However, they are moving more to iPads for operators.  Moving to 4 years is more reflective of reality.  1590 includes Flow Watch (Work and Info management) which is a cloud hosted system.  The capitalized cost is a 6-year asset.  Next biggest one is the integration software with UI – which will be replaced soon – more than 10 years old.  DW - Leaving at 8 years is a reasonable balance with the asset mix.

Transportation Equip WTR

1555 – includes small vehicles, large trucks, trailers, etc.  The current life is 5 years.  That is shorter than expected.  DW - Given the mix of assets (some only lasting 5 years and some lasting 10 years or longer), moving to a 7 years average life is reasonable.

Tools, Shop and Mis

1190 and 1470 both have a 10-year life currently.  DW - Based on the types of small assets in the account, 10 years is still a reasonable life.

Laboratory Equipment

1195 and 1475 (10 and 5 year lives, respectively currently).  Technology is causing the lives to decrease.  These are field-based labs.  DW - Moving consistently to 10 for both is reasonable.

Power Operated Equip

1200 and 1480 are currently 10 and 20 respectively. They maintain their equipment well.  DW - Moving to 20 consistently between both accounts is reasonable.

Communication Equip

1620 should all be retired and much of 1205 (related to Mitchell Co.).  The remaining is communication related to lift stations (e.g. auto dialers), etc. DW - Technology is changing but 10 years for this type of asset is reasonable.

Misc.

1210, 1490 and 1623 (10,66 and 10, respectively).  DW - They should all be 10 years based on the types of assets in the account (e.g. hand held readers).

Land/Land rights – do not depreciate