TO:		Phillip Lehmann, Attorney

Legal Division

FROM:	Patricia Garcia, Senior Engineering Specialist

Infrastructure Division

DATE:		May 17, 2021

RE:	Docket No. 51738 – Petition of Sanctuary Texas, LLC to Amend Aqua Texas Inc.’s Certificate of Convenience and Necessity in Denton County by Expedited Release

On January 20, 2021, Sanctuary Texas, LLC (Sanctuary) filed a petition for streamlined expedited release (SER) from Aqua Texas Inc.’s (Aqua Texas) water certificate of convenience and necessity (CCN) No. 13201 in Denton County, under Texas Water Code (TWC) § 13.2541(b) and 16 Texas Administrative Code (TAC) § 24.245(h). Sanctuary asserts that the land is at least 25 contiguous acres, is not receiving water service, and is located in Denton County, which is a qualifying county.

Sanctuary submitted a sworn affidavit attesting that the property was not receiving water service from Aqua Texas and a warranty deed confirming Sanctuary’s ownership of the tract of land. In addition, Sanctuary submitted sufficient maps and digital data for determining the location of the requested release area within Aqua Texas’s certificated service area.

On May 3, 2021, Aqua Texas filed a Response to Petition of the Sanctuary Texas, LLC for Streamlined Expedited Release.  In the response Aqua Texas stated that the Petitioner is in fact receiving water service and is not eligible for SER, as Aqua Texas has a water main line and meter located within the property.  Darryl Waldock, Aqua Texas’s Area Manager-North Texas, submitted an affidavit attesting to the following: (1) Aqua Texas has 2,930 linear feet of an active four-inch water main line located within the Property; (2) Aqua Texas has a residential 5/8” x ¾” meter located within the Property that until January 29, 2020 was used to actively supply water to a billed residential connection located at 601 Garza Lane, Little Elm, TX 75068-3509 and remains on standby today; and (3) Aqua Texas has other facilities immediately adjacent to the Property.  A map confirming the location of the water main and a previous bill for water service provided to 601 Garza Lane was submitted with the response.

In its petition, Sanctuary proposes to exclude from the released property the land on which the house located at 601 Garza Lane, Little Elm, TX 75068-3509 sits.  Sanctuary’s mapping of the area requested for release also excludes a 100 foot by 100 foot section that includes the water meter for the house and a 10 foot  wide by 570 foot long strip that runs over and around the water line that connects to the water meter.

Sanctuary’s proposal to exclude the strip of land running over and around the water line installed to serve the house at 601 Garza Lane is unnecessary and could cause confusion in the future about which utility is authorized to provide service as new structures needing water service are built on the property.  In addition, the Commission has left water lines certificated, while decertificating the land above, in previous cases like Docket No. 51114.  Therefore, I recommend that Sanctuary revise its mapping to restore the 10 foot by 570 foot strip such that the final map prepared by Staff reflect that the water line remains certificated and only the land above the water line is decertificated.  Making this change and leaving the 100 foot by 100 foot section that removes the water meter closely follows current Commission decisions.

As an alternative, I recommend that the water meter, like the water line, is eligible to remain certificated but that the land on which it sits should be decertificated.  TWC § 13.242(a) ties the right to serve granted by a CCN to the location of a “consuming facility” rather than the location of the facilities used to serve a consuming facility such as a water line and meter.  The Sanctuary’s mapping already excludes the land on which the house at 601 Garza Lane, Little Elm, TX 75068-3509 is located.  Therefore, the consuming facility that has been receiving water service from Aqua Texas, which is central to the TWC’s provision on the right to serve, is not located in the area to be released.  I recommend that the Commission reconsider the requirement to remove the land associated with a water meter that serves a consuming facility from an area to be released given that TWC § 13.242(a) ties the right to serve granted by a CCN to the location of a “consuming facility” and instead allows for the decertification of the land on which the water meter sits, while leaving the meter itself certificated, similar to how water lines have been treated.

Petition of Imperial Heights, Ltd. to Amend Aqua Texas, Inc.’s Certificates of Convenience and Necessity in Harris County by Expedited Release, Docket No. 51114, Order at Ordering Paragraph Nos. 1-2 (Dec. 17, 2020).