Phone (409)246-3463 Fax (409)246-2319

P.O. Box 188 Kountze, Tx 77625

October 13th, 2021

Public Utility Commission of Texas

Central Records

1701 N Congress, Suite 8-100

Austin, TX 78701

Re:  Project 52299, Water and Wastewater Critical Load Status for the City of Kountze water and wastewater facilities and City of Kountze - Ranchland water facilities

The following water and wastewater facilities qualify for critical load status.

- City of Kountze Water Facilities

- Feagin St (well #4) water well

Address – 1905 Feagin St., Kountze, TX 77625

Description – The Feagin water well is one of two sites that store and produce potable water, as well as maintain pressure, for the City of Kountze.  On site is a 600-gallon per minute well, treatment facilities, 2 700 gallon per minute booster pumps, and a ground storage tank.  A backup generator is onsite.

- Deer St  (well #5) water well

Address – 740 W Deer St., Kountze, TX 77625

Description – The Deer St water well is one of two sites that store and produce potable water, as well as maintain pressure, for the City of Kountze.  On site is a 600-gallon per minute well, treatment facilities, 2 600 gallon per minute booster pumps, and a ground storage tank.  A backup generator is onsite.

- Hart Ln elevated storage tower

Address – 100 Read St., Kountze, TX 77625

Description – The Hart ln EST is one of two sites that maintain the pressure for the City of Kountze.  This site also controls the pressure for the entire water system.  A pressure switch calls for the booster pumps at either the Feagin well or the Deer St well (alternates each time) whenever the pressure decreases under the desired set point.  There is no backup generator onsite.

- Redwood elevated storage tower

Address – 105 W Pope St., Kountze, TX 77625

Description – The Redwood EST is one of two sites that maintain pressure for the City of Kountze.  An altitude valve is in place to maintain the level in this EST.  The altitude valve is located in a vault that must be pumped down any time there is rainfall.  There is no backup generator onsite.

- City of Kountze Wastewater Facilities

- Wastewater treatment plant

Address – 199 Walters Ln., Kountze, TX 77625

Description – The wastewater treatment plant (WWTP) is the final treatment unit for the sanitary sewer system for the City of Kountze.  On site are pumps utilized to move the influent into the pond system, aerators to aid in the treatment process, chlorination facilities to remove E. coli before leaving the WWTP, and a flowmeter to calculate flows and control the chlorination equipment.  There is a backup generator onsite.

- Allen St liftstation

Location – 30.3946, -94.3312

Description – Liftstation responsible for moving untreated wastewater to the WWTP.  There is no backup generator onsite.

- Villa liftstation

Location – 30.381643, -94.314906

Description – Liftstation responsible for moving untreated wastewater to the WWTP.  There is a backup generator onsite.

- 418 liftstation

Location – 30.381474, -94.309071

Description – Liftstation responsible for moving untreated wastewater to the WWTP.  There is no backup generator onsite.

- Mobil liftstation

Location – 30.368332, -94.323858

Description - Liftstation responsible for moving untreated wastewater to the WWTP.  There is no backup generator onsite.

- Coe liftstation

Location – 30.360239, -94.312516

Description - Liftstation responsible for moving untreated wastewater to the WWTP.  There is a backup generator onsite.

- Spivey liftstation

Location – 30.336984, -94.309165

Description - Liftstation responsible for moving untreated wastewater to the WWTP.  There is a backup generator onsite.

- Forest Acres liftstation

Location – 30.360790, -94.306715

Description - Liftstation responsible for moving untreated wastewater to the WWTP.  There is no backup generator onsite.

- South Park liftstation

Location – 30.357608, -94.298830

Description - Liftstation responsible for moving untreated wastewater to the WWTP.  There is no backup generator onsite.

- City of Kountze – Ranchland Water Facilities

Address – 2250 Cypress Creek Trail

Description – The Ranchland water well is the only source of production and storage for the entire system.  On site is a 75-gallon per minute well, 900-gallon pressure tank, and treatment facilities.  A backup generator is on site.

- Emergency Contacts

Main point of contact – Tim Drake  office (409)246-3463, mobile (409)790-8994

Alternate contact – Josh Marble  office (409)246-3463, mobile (409)790-8994

Mailing Address:

The City of Kountze

PO Box 188

Kountze, TX 77625

If you have any questions please contact Tim Drake at The City of Kountze City Hall at 409-246-3463

Sincerely,

Tim Drake

Public Works Director