To Whom it Concerns,

Riverbend Water Resources District (RWRD) is a conservation and reclamation district that provides water and wastewater services to the Red River Army Depot and the TexAmericas Center industrial park with a 12,000-acre footprint. Riverbend formally represents, through Interlocal Agreements, the interests in water supply for the most northeast Texas region of the state representing the counties of Bowie, Cass and Red River, the Cities of Annona, Atlanta, Avery, DeKalb, Hooks, Leary, Maud, Nash, New Boston, Redwater, Texarkana (TX) and Wake Village, as well as Central Bowie County Water Supply Corporation, Oak Grove Water Supply Corporation, and TexAmericas Center. We are providing this letter with the information below to comply with Senate Bill 3 requirement for the November 1 due date.

Location of Facilities:

- Ron Collins Water Reclamation Facility (Domestic Wastewater Plant)

450 Hopkins Rd.

Hooks, TX. 75561

This is our general domestic wastewater plant that’s capable of providing treatment for monthly average flows of 1.5 million gallons per day and removes 90 percent of conventional pollutants from raw wastewater. The system can handle a daily maximum flow of 3 MGD.

- TexAmericas Center East Interconnect Booster Station

Located 208 yards southwest of Highway 82 and Bowie Parkway intersection at coordinates 33 degrees 27’42” N and 94 degrees 14’03” W.

This is a booster station the supplies water to the Tex-Americas Center East footprint that consist of approximately 8,400 acres. It has 2 high service pumps and one jockey pump to maintain pressure within the system currently.

- RWRD Ground Storage Tanks

Red River Army Depot

E Patrol Boundary Rd.

Hooks, TX. 75561

This facility has 2-500,000-gallon ground storage tanks with 4 high service pumps to provide water to a 500,000-gallon elevated tower on the west side of the Red River Army Depot. Between the ground storage tank and elevated tank, we maintain pressures all on the depot around 65-70 psi and provide an average daily flow of around 700,000 gallons to support the operations of Red River Army Depot.

- RWRD Industrial Wastewater Treatment Plant

Red River Army Depot

Combat Rd.

Hooks, TX. 75561

This is our industrial wastewater treatment plant that treats all process wastewater produce from the various operations performed by Red River Army Depot. The industrial plant can provide treatment for monthly average flows of .75 million gallons per day and removes 90 percent of conventional pollutants from raw wastewater. The system can handle daily maximum flows of 1.5 MGD.

- RWRD Elevated Water Tank

190 James Carlow Dr.

New Boston, TX. 75570

This is our 500,000-gallon elevated water tower that we use in conjunction with our ground storage tanks to help maintain pressure within our water distribution system.

Emergency Contact Information:

Eli Hunt – (903) 701-4916

Kyle Dooley – (903) 701-2218

Mailing Address:

Riverbend Water Resources District

228 Texas Ave., Suite A

New Boston, TX. 75570