City of Hondo Wastewater System

Permit Number:  WQ0010189001

Wastewater Treatment Plant

202 Plant Drive Hondo Texas 78861

This facility is located 1,400 feet east of the intersection of Farm-to-Market Road 462 and 30th Street

Lat. 29.3373	Long -99.1301

The treatment plant includes an influent lift station with 4 lift pumps, 2 bar screens, aeration basin with 2 aerators, 3 final clarifiers, U/V system, aerobic digester/sludge thickener (racetrack),1 belt filter press, and 24 drying beds. The facility has applied for critical Load Status and has a back-up diesel generator capable of running this facility for 30.6 hours under full load. The Treatment Plant is a 250 kilowatt, 480 voltage, 3 phase.

The current permit limits include an annual average flow of 1.8 million gallons per day and an average 2-hour peak flow of 4,167 gallons per minute (6.0 mgd.)

Prison Lift Station

597 PR 4303

This facility is located 5081 feet from the back gate of Hondo Airport located off Zachery Lane.

Lat. 29.3512	Long -99.1862

This lift station is a high flow lift station that includes 1 bar screen, 3 submersible pumps rated at 620 gpm. The lift station is listed under the critical load status with the application of the Wastewater Treatment Plant.  This lift Station has a back-up diesel generator capable of running this facility for 8.5 hours under full load. This stations is a 35 kilowatt, 480 voltage, 3 phase station.

STLT Lift Station

917A Carter St.

This facility is located 280 feet from the guard shack of South Texas Liquid Terminal.

Lat. 29.3457	Long -99.1300

This lift station is a low flow lift station that includes 2 submersible pumps rated at 100 gpm. The lift station is listed under the critical load status with the application of the Wastewater Treatment Plant.  This lift Station has a portable back-up diesel generator capable of running this facility for 7.5 hours under full load. This stations is a 31 kilowatt, 240 voltage, 3 phase station.

Ave K Lift Station

Ave K & 11th St.

This facility is located at the intersection of Ave K and 11th Street.

Lat. 29.3546	Long -99.1410

This lift station is a low flow lift station that includes 2 Centrifugal pumps rated at 200 gpm. The lift station is listed under the critical load status with the application of the Wastewater Treatment Plant.  This lift Station has a portable back-up diesel generator capable of running this facility for 7.5 hours under full load. This stations is a 31 kilowatt, 240 voltage, 3 phase station.