DOCKET NO. 52487

| APPLICATION OF ENTERGY TEXAS, INC. TO AMEND ITS CERTIFICATE OF CONVENIENCE AND NECESSITY TO CONSTRUCT ORANGE COUNTY ADVANCED POWER STATION | § § § § § | PUBLIC UTILITY COMMISSION OF TEXAS |
| --- | --- | --- |

DIRECT TESTIMONY

OF

NICHOLAS W. OWENS

ON BEHALF OF

ENTERGY TEXAS, INC.

SEPTEMBER 2021

ENTERGY TEXAS, INC.

DIRECT TESTIMONY OF NICHOLAS W. OWENS

DOCKET NO. 52487

TABLE OF CONTENTS

Page

I.	Introduction and Purpose	1

II.	The MISO Resource Adequacy Construct	5

III.	ETI’s Decision to Conduct Long-Term Generation Planning	16

IV.	Basis for ETI’s Long-Term Planning Reserve Margin Target	21

# Introduction and Purpose

PLEASE STATE YOUR NAME, BUSINESS ADDRESS, and position.

A.	My name is Nicholas W. Owens.  My business address is 30 Monument Square, Suite 105, Concord, Massachusetts 01742.  I am a Partner at The NorthBridge Group (“NorthBridge”).

ON WHOSE BEHALF ARE YOU TESTIFYING IN THIS PROCEEDING?

A.	I am submitting this Direct Testimony to the Public Utility Commission of Texas (“Commission”) on behalf of Entergy Texas, Inc. (“ETI or the “Company”).

PLEASE DESCRIBE YOUR EDUCATIONAL BACKGROUND AND PROFESSIONAL EXPERIENCE.

    - I graduated from Colby College in 2004 with a B.A. in economics and government.  I spent two years as an Analyst with FTI Consulting before joining NorthBridge in 2007.  I became a Partner at NorthBridge in 2019.  NorthBridge is an economic and strategic consulting firm serving the electricity and natural gas sectors.  My practice includes generation planning and operations within regional transmission organization (“RTO”) markets.

HAVE YOU PREVIOUSLY PROVIDED TESTIMONY BEFORE A REGULATORY COMMISSION?

A.	Yes.  I provided testimony to the Louisiana Public Service Commission in Docket Nos. U-32148 and U-33592 and to the City Council of New Orleans in Docket No. UD-11-01.

what is the purpose of your testimony?

A.	I am testifying on behalf of the Company in support of its application seeking an amendment to its certificate of convenience and necessity to construct, own, and operate the Orange County Advanced Power Station (“OCAPS” or the “Project”).  Specifically, my testimony: 1) describes the Midcontinent Independent System Operator, Inc. (“MISO”) annual capacity auction, which is called the Planning Resource Auction (“PRA”), and the limited role that it plays in ensuring resource adequacy within MISO; 2) explains the need for Load Serving Entities (“LSEs”) in MISO to conduct long-term generation planning and to support enough generation to meet their expected capacity needs primarily through ownership and/or bilateral contracts, rather than through reliance on the PRA; 3) explains the reasonableness of ETI’s decision to value OCAPS by comparing it to the cost of an alternative source of real, physical capacity in East Texas, rather than a projection of the PRA price; and 4) explains the basis for ETI’s long-term planning reserve margin target.

please summarize your conclusions.

A.	First, the MISO PRA is not like capacity markets in the Eastern RTOs (PJM, NYISO, and ISO-NE), which are characterized mostly or entirely by retail access and rely on their capacity markets as the primary means to maintain resource adequacy.  Unlike these eastern capacity markets, and as MISO itself has acknowledged, the PRA is not designed to be a substitute for long-term resource planning.  By virtue of its design, it does not send efficient long-term price signals for generation entry or exit and will not retain and/or bring forth enough new capacity to maintain resource adequacy.  Rather than rely on the PRA to maintain resource adequacy, MISO (like the Southwest Power Pool (“SPP”)) relies on its LSEs, which consist largely of vertically integrated public utilities regulated at the state level, to engage in reasonable long-term generation planning, to plan enough capacity to meet their expected needs, and to support the construction and ongoing operation of that generation by making long-term financial commitments through ownership and/or bilateral contracts.

Second, while it may on the surface appear attractive for ETI to plan to meet its expected capacity needs by “leaning” on the PRA, that would be a poor strategy involving substantial risk to ETI customers over the long-term.  In recent certification proceedings, intervenors have pointed to persistently low capacity prices in the PRA to suggest that it would be less costly for ETI to plan to purchase capacity in the PRA than to add new long-term generation in its Texas service territory.  It is true that PRA prices have been low, but ETI should not abandon long-term planning.  ETI can and does reasonably avail itself of purchases in the annual PRA to balance near-term capacity deficits.  However, as noted above, the PRA is not a substitute for long-term planning; it is not designed to send price signals to induce new entry where and when necessary to maintain reliability.  If ETI were to deactivate a significant portion of its aging generation in East Texas, as it is reasonably planning to do, and lean primarily or solely upon the PRA to replace that capacity, it would be unreasonable to expect and rely upon merchant entry to materialize in East Texas as a means of maintaining reliable and economic service to ETI’s customers.  Rather, under these circumstances, there would be a significant reduction of capacity inside the load pocket in which ETI’s service territory is located, and therefore, a higher risk of local scarcity events, a higher risk of involuntary load shed, and ETI would have a much larger net short position and more exposure to price spikes than it already does.  These are not abstract concerns.  ETI experienced local shortages twice during the past year following Hurricane Laura and Winter Storm Uri and was exposed to extremely high spot prices and forced to shed firm customer load during both events.  In addition, MISO is proposing changes to its resource adequacy construct that would impose new restrictions to prevent LSEs from relying too heavily on the PRA to meet their expected capacity needs and impose penalties upon those who do.

Third, ETI has reasonably chosen to value OCAPS by comparing the cost of OCAPS to the cost of an alternative source of real, physical capacity (in this case, as measured by the cost of a new combustion turbine), rather than a projection of the PRA price.  This makes sense because, consistent with the MISO resource adequacy construct, ETI has reasonably chosen to meet its expected capacity needs through ownership of and bilateral contracts with long-term resources.  It would not make sense to compare the cost of OCAPS to the cost to purchase capacity in the PRA because ETI has reasonably decided not to rely on PRA purchases as a long-term strategy.  Further, such a comparison would not make sense because PRA purchases are not comparable to physical capacity; in particular, unlike physical capacity, PRA purchases do not contribute to local reliability or hedge exposure to energy prices.

Fourth, ETI’s long-term planning reserve margin target reflects the amount of capacity that must be planned to achieve resource adequacy.  The planning reserve margin required to achieve resource adequacy increases as a function of uncertainty in the load forecast, and uncertainty in the load forecast increases as a function of the length of the forecast.  Because it takes approximately four years to plan a new resource, ETI’s long-term planning reserve margin target is reasonably calculated using an estimate of uncertainty in the four-year-ahead load forecast.

# The MISO Resource Adequacy Construct

DOES MISO HAVE A TARGET planning RESERVE MARGIN?

A.	Yes.  Although MISO does not establish a long-term planning reserve margin requirement for LSEs within its footprint, it does establish such a target planning reserve margin on an annual basis, and LSEs must procure sufficient capacity each year to comply with that target.  MISO’s target planning reserve margin is expressed as a percentage of the MISO-coincident peak load forecast for the upcoming delivery year and is calculated such that, on an expected basis, there is a 10% probability that firm load will need to be curtailed in the upcoming year.  For example, MISO’s target planning reserve margin for the June 2021-May 2022 delivery year is 9.4%.  This means that if the forecast of MISO’s coincident peak load for the upcoming year and the amount of capacity in MISO were 100 GW and 109.4 GW, respectively, then MISO would expect the probability of firm load curtailment to be 10%.  A 10% probability of having to curtail firm load in each year translates to an expectation that firm load curtailment will occur once every ten years.  This “1-in-10” standard is commonly used to quantify the amount of capacity that would be adequate to serve customer needs.

DOES MISO CONDUCT A CAPACITY AUCTION?

A.	Yes.  MISO conducts an annual capacity auction – the PRA – in the spring of each year to commit capacity for the upcoming June-May delivery period.  Demand in the auction is equal to the sum of each LSE’s MISO-coincident peak load forecast plus the MISO target planning reserve margin.  Supply in the auction is represented by offers from generators to provide capacity, which entails an obligation to submit an offer in the energy market on each day of the delivery period that the generator is not on an outage.

The annual auction includes constraints that represent the minimum amount of capacity deemed necessary to maintain resource adequacy in each of ten zones for the upcoming planning year.  These zones are referred to as Local Resource Zones (“LRZ”), and the minimum amount of capacity required in each zone is referred to as that zone’s Local Clearing Requirement (“LCR”).  ETI’s service territory is located within LRZ 9, which incorporates the Texas and Louisiana portions of MISO.  In addition to the minimum requirements for each LRZ, the auction also includes constraints to commit a minimum amount of capacity in the two subregions of MISO – MISO South and MISO North/Central.

MISO clears the auction with an objective to minimize the as-offered cost of cleared capacity, subject to constraints that ensure enough capacity is cleared in each of the 12 zones and in aggregate to meet the target planning reserve margin.  The auction results include market clearing prices for each zone, which may be different if locational constraints are binding.  In particular, the price in a zone may be higher than in other zones if a higher price in that zone is necessary to clear enough capacity to meet the applicable local clearing requirement.

Prices are capped at MISO’s estimate of the levelized cost of a combustion turbine, also referred to as the Cost of New Entry (“CONE”).  Each generator that clears the auction is paid the market clearing price of the zone in which the generator is located.  The payments to generators are funded by charges assessed to LSEs, which are based on the price of the zone in which the LSE serves customers.  Figure 1 includes a map of the zones and the price for LRZ 9 during the past eight years.  As can be seen from the figure, the price has been very low – approximately $6/MW-day on average.  For reference, MISO has estimated that the levelized cost of a new combustion turbine, or CONE, in LRZ 9 is approximately $240/MW-day.

# Figure 1

Local Resource Zones and PRA Clearing Prices for LRZ 9

DO MISO PRA PRICES REFLECT THE COST OF CAPACITY?

A.	No.  The PRA is conducted less than three months prior to the beginning of the delivery year.  At the time offers are due, most generators have already incurred the costs and/or committed to incur the costs necessary to operate during the delivery year.  These “sunk” costs include the cost of construction, maintenance, staffing, etc.  The incremental costs to provide capacity are very low at this time, and thus most generators submit offers to provide capacity at or close to $0, as can be seen in Figure 2 below.  During the most recent auction for the 2021/2022 delivery year, 92% of capacity was offered at $0, 95% was offered below $1/MW-day, and 98% was offered below 10% of CONE.  With so many low-priced offers, which are rational given when the auction is conducted relative to the delivery year, it is not surprising that PRA prices have been so low.

# Figure 2

Supply and Demand in the MISO PRA for Delivery Year 2021/2022

COULD PRA PRICES increase?

A.	Yes.  Both supply and demand in the PRA are inelastic, and this means that even modest changes in supply or demand could cause prices to rise abruptly.  The demand curve is vertical, and thus is completely inelastic.  The relevant portion of the supply curve is nearly vertical because the auction is conducted less than three months before the delivery year, as discussed above, and thus is extremely inelastic.  Figure 2 shows that a net change in the supply-demand balance of less than 5% would leave MISO without enough capacity to meet the target planning reserve margin, in which case the market clearing price would be set administratively at CONE, the cap.

is the pra designed to ensure resource adequacy?

A.	No.  The design of the PRA is such that prices will nearly always be very low or set at the CONE price cap.  To an investor considering the construction of a merchant power plant, the PRA is very unattractive.  In some years, the price may be set at the levelized cost cap and in other years the price will be close to zero, such that the average price over time will be less than the levelized cost, i.e., less than necessary to recover a return of and on the investment.  This phenomenon is well understood.  For example, in testimony filed in support of a proposal by MISO to modify the PRA, MISO’s consultant, the Brattle Group, offered the following conclusion:

We find that MISO’s current capacity market design is unlikely to attract and retain sufficient merchant capacity to meet MISO’s 1-in-10 reliability standard in the long-term.  This is because the combination of a vertical demand curve, inelastic prompt supply curve, and low price cap will produce prices that are volatile and capped at a moderate or low level.  Prices will not rise high enough on average to support new entry until the reserve margin is low enough that the market is facing frequent shortages.

DO OTHER RTOS RELY ON CAPACITY MARKETS TO ENSURE RESOURCE ADEQUACY?

A.	Yes.  The three Eastern RTOs (PJM, NYISO, and ISO-NE) are all characterized primarily or entirely by retail access and rely on their capacity markets to ensure resource adequacy.  Most of the utilities participating in these RTOs have been restructured, meaning that they have divested their generation, are no longer vertically integrated, and no longer conduct generation planning.  Because these utilities no longer conduct generation planning, these RTOs rely on merchant generators to make efficient decisions – in response to wholesale price signals – to continue operating, to retire, and to build new capacity in the right location and at the right time.  To create the price signals necessary to promote efficient decisions, these RTOs have carefully designed their capacity markets to incorporate features lacking in the PRA.  Notably, the Eastern RTOs have capacity markets that feature sloping demand curves, higher price caps, and forward procurement, as described by MISO’s consultant, the Brattle Group.

These problems have been addressed in other capacity markets that rely on merchant supply resources.  All capacity markets other than MISO have improved price stability through sloped demand curves, while also incorporating higher price caps to support reliability… both PJM and ISO-NE further mitigate capacity market price volatility and provide additional forward certainty by holding forward capacity auctions.  [emphasis added]

The MISO PRA does not include any of these features, which are overwhelmingly opposed by MISO stakeholders.  MISO has no plans to propose them, and past attempts to impose them have been rejected by FERC, which reasoned that MISO has a different approach to resource adequacy.,

WHAT IS MISO’s approach to resource adequacy?

A.	Rather than rely on the PRA to ensure resource adequacy, MISO relies on LSEs to engage in long-term generation planning.  According to MISO, LSEs “bear responsibility for ensuring resource adequacy.”  This means that LSEs, subject to the oversight of their retail regulators, such as ETI, are responsible for planning enough generation to meet their expected needs and for supporting the construction and ongoing operation of that generation by making long-term financial commitments through ownership and/or bilateral contracts.  LSEs that do not plan enough generation to meet their expected needs and instead plan to purchase capacity from the PRA at prices that do not reflect the cost of capacity are “leaning” on others to do so for them.  The MISO Tariff does not currently prohibit leaning, but it would be unwise to pursue that as a strategy, as discussed below.

IF THE PRA IS NOT DESIGNED TO ENSURE RESOURCE ADEQUACY, WHAT IS THE PURPOSE OF THE PRA?

A.	While MISO relies on LSEs to plan enough generation to meet their expected capacity needs, its resource adequacy construct recognizes that some LSEs will have a surplus and others will have a shortfall on a short-term basis.  This may be due to the lumpiness of capacity additions, the need to accelerate a unit retirement, differences between actual and expected demand, etc.  The PRA allows LSEs to “balance” their capacity positions on a short-term basis; those with a surplus may sell their excess and those with a shortfall may purchase their residual needs.  This balancing function of the PRA can be helpful because it allows market participants to transact on a short-term basis without the “friction” associated with bilateral contracting.

HOW DOES MISO’s RESOURCE ADEQUACY CONSTRUCT COMPARE TO SPP’S?

A.	They are similar.  Like MISO, SPP relies on each LSE to support enough capacity to meet its needs through ownership or bilateral contracts.  Unlike MISO, SPP does not operate a capacity auction; its tariff requires each LSE to provide enough capacity through ownership or bilateral contracts to meet 100% of its peak load plus the SPP planning reserve margin, and it has stiff penalties for LSEs that fail to make a sufficient showing.  In the most recent showing for the summer of 2021, all 58 LSEs in SPP made a sufficient showing.

HOW DOES MISO’s RESOURCE ADEQUACY CONSTRUCT COMPARE TO ERCOT’S?

A.	They are different.  Unlike MISO, ERCOT is characterized mostly by retail access.  The investor-owned utilities in ERCOT have been restructured and no longer conduct generation planning.  ERCOT is more like the Eastern RTOs in that it relies on merchant generators acting in response to wholesale price signals to maintain resource adequacy.  But rather than use a carefully-designed capacity market to create the wholesale price signals necessary to maintain resource adequacy, ERCOT uses a carefully designed mechanism to create scarcity prices in the markets for energy and ancillary services – i.e., the operating reserve demand curve.

Figure 3 summarizes the composition of these markets and their approach to maintaining resource adequacy.  As shown in Figure 3, the RTOs where the majority of load is served by competitive suppliers (i.e., the Eastern RTOs and ERCOT) rely on price signals to ensure resource adequacy, while the RTOs where the majority of load is served by vertically integrated utilities, municipals, and cooperatives (i.e., SPP and MISO) rely on LSEs (and, in the case of regulated LSEs, the oversight of their retail regulators) to ensure resource adequacy by engaging in long-term planning.

# Figure 3

Market Composition and Approach to Resource Adequacy

HAS MISO PROPOSED ANY CHANGES TO ITS RESOURCE ADEQUACY CONSTRUCT?

A.	MISO has proposed two notable changes.  First, MISO has proposed to change from an annual to a seasonal construct.  This entails the creation of a separate planning reserve margin and capacity requirement for each of the four seasons, generator ratings for each season that reflect seasonal availability, and a capacity auction for each season to clear supply and demand.  While the seasonal proposal will change the PRA, it will not introduce the eastern-style capacity market features – sloping demand curve, higher price caps, and forward procurement – that would be necessary to ensure resource adequacy in the absence of vertical integration, as I discussed above.  Therefore, it will not affect MISO’s fundamental reliance on LSEs to plan enough generation to meet their expected needs.

Second, consistent with the “fundamental assumption” underlying the MISO resource adequacy construct that LSEs are engaging in reasonable long-term resource planning, MISO has proposed to require each LSE to demonstrate prior to the PRA that it has self-supplied through ownership or bilateral contract at least 50% of its capacity requirement in all four of the upcoming seasonal delivery periods.  As with the seasonal proposal, this proposal to create a Minimum Capacity Requirement (“MCR”) does not affect MISO’s fundamental reliance on LSEs to plan enough generation to meet their expected needs; indeed, it makes such reliance more explicit in MISO’s market rules.

# ETI’s Decision to Conduct Long-Term Generation Planning

Is ETI’s decision TO PLAN ENOUGH GENERATION TO MEET ITS expected CAPACITY NEEDS CONSISTENT WITH THE MISO RESOURCE ADEQUACY CONSTRUCT?

A.	Yes.

BUT TECHNICALLY, COULD ETI PLAN TO LEAN ON THE PRA?

A.	That would be inconsistent with the MISO resource adequacy construct, but the MISO Tariff does not currently prohibit ETI from leaning on the PRA.  As noted above, MISO has proposed to prohibit an LSE from leaning on the PRA to meet more than 50% of its MISO capacity requirement.

PRA PRICES HAVE BEEN VERY LOW.  WHY NOT LEAN ON THE PRA?

A.	There are several good reasons not to do that.

First, PRA prices have been low, but that does not mean that they will stay low.  As discussed above, PRA prices can rise abruptly given the extreme inelasticity of both supply and demand.  Recent low PRA prices would seem to indicate that MISO is awash in surplus capacity, but that is not the case.  Figure 2 shows that a net change in the supply-demand balance of less than 5% would leave MISO without enough capacity to meet its target planning reserve margin, in which case the clearing price would be set at CONE, the cap.  With the number of aging resources in MISO and the pressure to retire those resources, including coal, a 5% reduction in the supply-demand balance is certainly possible by the time OCAPS is planned to be in-service five years from now, in 2026.  This would not represent an extraordinary shift.  For example, the ERCOT planning reserve margin fell by 7.6% in one year between 2017 and 2018.

Second, if ETI were to deactivate a significant portion of its aging generation in East Texas, it is very unlikely that merchant entry would occur in East Texas to replace it.  As discussed above, the PRA is not designed to be a substitute for long-term planning; it lacks the design features necessary to send efficient price signals for merchant entry or to provide sufficient revenues over the life cycle of a new resource to cover the cost of ownership, including a reasonable return on investment.  Thus, unless ETI plans to replace its aging local generation with new local generation, there would be a reduction of capacity within the ETI service territory and a degradation of reliability.

ETI’s service territory is located entirely within the West-of-the-Atchafalaya-Basin (“WOTAB”) load pocket, which is not supported by flows from the west because of the asynchronous border with ERCOT or from the south because of the Gulf of Mexico.  ETI relies on local generation and a finite amount of transmission from the east to serve its customers, as shown in Figure 4.  A reduction in local generation would increase the expected frequency of local scarcity events and reduce the reliability of service to ETI’s customers.  These are not abstract concerns – ETI experienced local scarcity twice during the past year following Hurricane Laura and Winter Storm Uri and was forced to curtail firm service to its customers during both events.  New local generation can mitigate the risk of such events, but that generation is very unlikely to materialize if ETI leans on the PRA.  Even if merchant generation were to be built in East Texas, there is no certainty it would be built in time to timely or adequately protect ETI customers from generation scarcity.  ETI witness Daniel Kline’s Direct Testimony further describes the transmission-related benefits of local generation.

# Figure 4

ETI Relies on Local Generation and Finite Transmission Capacity from the East

Third, if ETI were to deactivate its aging resources and lean on the PRA to meet its expected capacity needs instead of bringing online actual energy-producing assets, that would aggravate ETI’s and its customers’ net exposure to energy purchases at the Locational Marginal Price (“LMP”).  ETI purchases 100% of its energy needs from the MISO market at LMP, but the cost of these purchases is offset by the revenue derived from the sale of energy from owned and contracted generation at LMP.  Unlike owned and contracted generating capacity, capacity purchases in the PRA do not convey a right to energy and therefore would not hedge ETI’s exposure in the energy market.  A deliberate plan to rely on the PRA to meet a substantial portion of ETI’s capacity needs would not only entail exposure to prices in the PRA, which are volatile by design, but also to volatile energy prices.  As discussed in the Direct Testimony of ETI witness Abigail Weaver, in the absence of OCAPS, ETI is projected to be short almost 9.2 TWh, or nearly 40% of the energy needed to serve ETI’s customers.  A long-term strategy to purchase this much energy from the MISO spot market would entail substantial risk for ETI’s customers.

HAS ETI REASONABLY DECIDED TO PLAN ENOUGH GENERATION TO MEET ITS EXPECTED CAPACITY NEEDS, RATHER THAN PLAN TO RELY ON THE PRA?

A.	Yes.  ETI’s decision is not only consistent with the MISO resource adequacy construct, which relies on LSEs to plan this way, but also entirely consistent with ETI’s obligation to provide service to its customers in a reliable and economic manner while managing risk.  Further, it is consistent with the planning practices of virtually every other vertically integrated utility in MISO.  I am unaware of any other large vertically integrated utility in MISO that plans to meet a substantial portion of its expected capacity needs through reliance on the PRA.  If ETI were to pursue such a strategy, it would be alone in doing so.

given eti’s decision TO PLAN ENOUGH GENERATION TO MEET ITS expected CAPACITY NEEDS, is it reasonable for eti to ESTIMATE the BENEFITS OF OCAPS by comparison to the cost of a real, physical capacity alternative?

A.	Yes.  This makes sense because ETI has reasonably chosen to meet its expected capacity needs primarily through ownership and bilateral contracts and, again, because PRA purchases are not comparable to physical capacity.  If ETI were not planning OCAPS, it would plan to meet its needs through ownership of or a bilateral contract with another resource, and the cost of a combustion turbine is a reasonable proxy for the cost of that alternative.  It would not make sense to compare the cost of OCAPS to PRA prices because ETI is not pursuing that as a long-term strategy, for good reasons.

# Basis for ETI’s Long-Term Planning Reserve Margin Target

DOES ETI have A LONG-TERM PLANNING RESERVE MARGIN target?

A.	Yes.  ETI plans to have approximately enough capacity to meet its forecast of expected coincident peak load plus a 12.69% long-term planning reserve margin.  ETI’s long-term planning reserve margin is higher than the short-term planning reserve margin published by MISO, which is 9.4% for the June 2021-May 2022 delivery year.

WHY IS ETI’S LONG-TERM planning RESERVE MARGIN HIGHER THAN MISO’S SHORT-TERM planning RESERVE MARGIN?

A.	Both planning reserve margins are estimates of the amount of capacity, above and beyond the forecast of coincident peak load, that would be necessary to meet the “1-in-10” standard.  In other words, both planning reserve margins represent the amount of capacity needed to ensure that, on an expected basis, firm load would be curtailed only once every ten years.

The planning reserve margin required to meet the 1-in-10 standard is a function of uncertainty in three variables – generator availability, weather-driven load levels, and the weather-normalized load forecast.  As illustrated in Figure 5, higher uncertainty in any of these variables would translate to a higher planning reserve margin.

# Figure 5

Higher Uncertainty Translates to a Higher Planning Reserve Margin

ETI’s long-term planning reserve margin is higher than MISO’s short-term planning reserve margin because there is more uncertainty in the third variable – that is, the long-term weather-normalized load forecast is more uncertain than the short-term weather-normalized load forecast.  That there is more uncertainty in the long-term forecast is intuitive – the forecast is more uncertain the further into the future one looks.

MISO’s short-term planning reserve margin, on the other hand, represents the amount of capacity that must be in-service during the upcoming delivery year, and it therefore reflects the relatively small amount of uncertainty that exists in the one-year-ahead forecast.  In contrast, it takes approximately four years between when a new resource is proposed and when it comes online, so ETI’s long-term planning reserve margin, which reflects the amount that must be planned to maintain resource adequacy, reflects the relatively large amount of uncertainty that exists in the four-year-ahead forecast.  Thus, the two planning reserve margins are conceptually consistent and aligned.

DOES this conclude your direct testimony?

A.	Yes, at this time.

If ETI were to deactivate Sabine Units 1, 3, and 4, which units would be 64, 60, and 52 years old in 2026, and rely on the PRA to replace that capacity rather than build OCAPS, ETI estimates that its projected net short position for capacity and energy would be approximately 25% and 40%, respectively.

Testimony of Dr. Samuel A. Newell, Dr. Kathleen Spees, and Dr. David Luke Oates on Behalf of the Midcontinent Independent System Operator Regarding the Competitive Retail Solution at 4, FERC Docket No. ER17-284 (filed Nov. 1, 2016).

Id. at 11.

“The vast majority of MISO States do not depend solely on market price signals for generation resource investment.  Traditionally-regulated States have successfully ensured, through integrated resource planning, that their load serving entities (“LSEs”) have sufficient capacity to meet load obligations.  To the extent additional capacity is necessary, LSEs subject to state supervision and authority have a clearly defined process for building new generation and investing in existing generation resources.  Consequently, no changes to the current PRA construct are warranted for States that regulate utilities on a traditional basis.”  MISO Transmittal Letter for Competitive Retail Solution Filing at 4-5, FERC Docket No. ER17-284 (filed Nov. 1, 2016).

“As an initial matter, we note that many of the arguments raised on rehearing seek to impose on MISO the rules and requirements used in the centralized capacity markets in the Eastern Regional Transmission Organizations (“RTO”)/Independent System Operators (“ISO”).  As discussed below, we reject those arguments, recognizing the regional differences between MISO and those RTOs/ISOs… unlike the centralized capacity constructs used in the Eastern RTOs/ISOs, MISO’s Auction is not – and has never been – the primary mechanism for its LSEs to procure capacity.”  Order Denying Rehearing at P 13, FERC Docket No. ER18-462-001, (filed Mar. 20, 2020) (citations omitted, emphasis in original).

“A fundamental assumption of the RA construct is that all LSEs are appropriately planning long-term.”  RAN Reliability Requirements and Sub-Annual Construct, a presentation by MISO to the Resource Adequacy Subcommittee at 26 (Mar. 10, 2021).

“The majority of States representing 90 percent of load in the footprint operate under a traditional regulatory structure.  In these areas, Load Serving Entities (“LSEs”), with oversight by State regulatory authorities, bear responsibility for ensuring resource adequacy.”  Prepared Direct Testimony of Richard Doying on behalf of MISO at 3, FERC Docket No. ER17-284 (filed Nov. 1, 2016).

“The capacity auction is designed to balance the market, not be a primary source of capacity while sending new build price signals.”  RAN Reliability Requirements and Sub-Annual Construct, a presentation by MISO to the Resource Adequacy Subcommittee at 13 (May 12, 2021).

SPP Open Access Transmission Tariff, Attachment AA.

2021 SPP Resource Adequacy Report at 5 (June 15, 2021).

See note 6, supra.

Resource Adequacy Reforms, a presentation by MISO to the Resource Adequacy Subcommittee at 30-33 (Aug. 4, 2021).

“MISO recognizes that participation in the PRA is not a substitute for long-term resource planning, a primary responsibility held by LSEs with applicable state jurisdictional oversight.  In MISO, LSEs must demonstrate resource adequacy and the MCR would be an additional step in that demonstration.”  Resource Adequacy Reforms, a presentation by MISO to the Resource Adequacy Subcommittee at 9 (June 9, 2021).

The planning reserve margin fell 7.6% from 16.9% in 2017 to 9.3% in 2018.  It fell a further 1.2% to 8.1% in 2019.  2016 ERCOT Capacity, Demand and Reserves (“CDR”) Report at 9 (Dec. 15, 2016); 2017 ERCOT CDR Report at 9 (Dec. 18, 2017); 2018 ERCOT CDR Report at 9 (Dec. 4, 2018).

While renewables have some limitations in replacing capacity, the timeframe for approvals and construction of new solar resources has been approximately 4 years.