Greasewood II LLC

Low Impact CIP Physical Security Plan

| Revision | Revision Date | Summary of Changes | Prepared By |
| --- | --- | --- | --- |
| v1 | 12/15/2023 | Initial Version | Electric Power Engineers (EPE) |

Table of Contents

1. Purpose	3

2. Applicability	3

3. Responsibilities	3

4. Procedure Details	3

Access Authorizations	3

Primary and Ancillary Physical Security Controls	4

5. Glossary of Terms	6

12. Reference Documents	7

# 1. Purpose

The purpose of this procedure is to provide Greasewood II LLC (Greasewood) written guidance on how to comply with NERC Standard CIP-003 Attachment 1, Section 2 for their Low Impact BES Cyber Systems.

This procedure shall be used to document Greasewood Physical Security Plan and Visitor Control process providing procedural controls to restrict physical access to identified BES Cyber Assets.

# 2. Applicability

Greasewood’s low impact BES Cyber Systems and associated BES Cyber Assets are subject to the processes herein.

# 3. Responsibilities

The CIP Senior Manager or designee shall ensure that physical security controls are documented and implemented at Greasewood and identified secured BES Cyber Asset rooms/areas.

To maintain reliability, Greasewood personnel are responsible for the maintenance and implementation of primary physical controls such as:

- Physical site boundaries

- Keyed access

- Badge readers

Note that additional physical access controls may be utilized to augment Greasewood’s defense-in-depth strategy.

# 4. Procedure Details

Always notify site management immediately of any violation or suspected violation of Greasewood’s Physical Security Plan. (See FRM-Physical Security Incident Investigation for reporting steps).

## Access Authorizations

Greasewood must maintain documentation of personnel physical access authorizations. Access authorization to the facility is provided by the Site Manager or designee.

The CIP Senior Manager provides physical access authorizations to secured BES Cyber Assets rooms/areas. Tailgating anywhere in or around the facility is strictly prohibited.

Non-Employee Access Restrictions

Contractors will, upon arrival:

- check in with appropriate plant personnel.

- be instructed on Greasewood’s physical access requirements.

Note: Certain Contractors, excluding Visitors, may be granted unescorted access to devices used by the site for electronic access control per CIP-003 or to other areas that contain BES Cyber Systems.

Visitors will, upon arrival:

- check in with appropriate plant personnel.

- be instructed on Greasewood’s physical access requirements.

Note: Visitors are restricted from entering secure BES Cyber Asset rooms/areas unless being continuously escorted. During emergency scenarios, unescorted access permissions may be granted to Visitor(s) based on safety needs. (See Cyber Security Incident Response Plan for emergency response and CIP Exceptional Circumstance guidance)

## Primary and Ancillary Physical Security Controls

Greasewood maintains the following primary physical controls:

| Control | Description |
| --- | --- |
| Perimeter Fencing | 6’ Cattle fencing and 6’ barbed wire topped fence around substation. |

Additional defense-in-depth security controls include:

| Control | Description |
| --- | --- |
| Hard Keys | Physical keys are used to provide an additional level of security and prevent unauthorized access to and/or within the facility |

Perimeter Fencing

- Fencing is 6 feet tall, topped with barbed wire, surrounding all sides of the facility.

- There is multiple gate access points, each with key controlled access.

- All perimeter fencing is inspected as part of periodic walk down conducted by plant personnel. Plant personnel check to make sure that the fence is in good working condition and no damage, tampering, attempted subversion, or unauthorized access has occurred.

Any damage, tampering, attempted subversion, or unauthorized access to the perimeter fencing identified during the walk down is to be reported to the control room/facility management. (See FRM-Physical Security Incident Investigation)

Hard Key Management

- Distribution of hard keys and the employees and/or contractors responsible for those keys are managed in a dedicated repository.

- Hard keys are collected upon termination of employment or upon a determination that the employee or contractor no longer needs unescorted access to the facility (e.g., retirement, termination, job transfer).

- The employee and/or contractor to whom the key is assigned holds the ultimate responsibility for the key.

- Lost keys must be immediately reported to the control room or Operations Manager.

- Plant personnel utilize a logbook for checking out/checking in keys whenever a physical key is needed to perform work at the facility.

- The following areas are secured via hard keys:

  - Perimeter fence gates

  - Control house

# 5. Glossary of Terms

Bulk Electric System (BES)

See NERC “Glossary of Terms Used in NERC Reliability Standards” definition.

BES Cyber Asset (BCA)

A Cyber Asset that if rendered unavailable, degraded, or misused would, within 15 minutes of its required operation, misoperation, or non‐operation, adversely impact one or more Facilities, systems, or equipment, which, if destroyed, degraded, or otherwise rendered unavailable when needed, would affect the reliable operation of the Bulk Electric System. Redundancy of affected Facilities, systems, and equipment shall not be considered when determining adverse impact. Each BES Cyber Asset is included in one or more BES Cyber Systems.

BES Cyber System (BCS)

One or more BES Cyber Asset logically grouped by a responsible entity to perform one or more reliability tasks for a functional entity.

CIP Exceptional Circumstance

A situation that involves or threatens to involve one or more of the following, or similar, conditions that impact safety or BES reliability: a risk of injury or death; a natural disaster; civil unrest; an imminent or existing hardware, software, or equipment failure; a Cyber Security Incident requiring emergency assistance; a response by emergency services; the enactment of a mutual assistance agreement; or an impediment of large-scale workforce availability.

CIP Senior Manager

A single senior management official with overall authority and responsibility for leading and managing implementation of and continuing adherence to the requirements within the NERC CIP Standards, CIP-002 through CIP-011.

Cyber Asset

Programmable electronic devices, including the hardware, software, and data in those devices.

Removable Media

Storage media that (i) are not Cyber Assets, (ii) are capable of transferring executable code, (iii) can be used to store, copy, move, or access data, and (iv) are directly connected for 30 consecutive calendar days or less to a BES Cyber Asset, a network within an ESP, or a Protected Cyber Asset. Examples include, but are not limited to, floppy disks, compact discs, USB flash drives, external hard drives, and other flash memory cards/drives that contain nonvolatile memory.

Transient Cyber Asset (TCA)

A Cyber Asset that:

- is capable of transmitting or transferring executable code,

- is not included in a BES Cyber System,

- not a Protected Cyber Asset (PCA) associated with high or medium impact BES Cyber Systems, and

- directly connected (e.g., using Ethernet, serial, Universal Serial Bus, or wireless including near field or Bluetooth communication) for 30 consecutive calendar days or less to a:

- BES Cyber Asset,

- network within an Electronic Security Perimeter (ESP) containing high or medium impact BES Cyber Systems, or

- PCA associated with high or medium impact BES Cyber Systems.

- Examples of Transient Cyber Assets include, but are not limited to, Cyber Assets used for data transfer, vulnerability assessment, maintenance, or troubleshooting purposes.

NERC Glossary of Terms

https://www.nerc.com/files/glossary_of_terms.pdf

# 12. Reference Documents

FRM-Physical Security Incident Investigation

Cyber Security Incident Response Plan