CIP-003

Physical Security Controls Plan

- Objective

  - This document provides the CIP Cyber Security Physical Security Controls Plan for Elawan Energy’s low impact BES Cyber Systems. This procedure is used as a part of the compliance requirements outlined in NERC Reliability Standard CIP-003.

- Applicable Systems

  - The purpose of this plan is to detail the physical security controls implemented to enhance the security of the BES Cyber Systems based on Elawan Energy Applicable Systems.

- Terms and Definitions - Terms defined in the NERC Glossary of Terms are capitalized and used as defined there. The additional terms listed in this section are specific to this document.

  - Access Device- Any card, key, code, or other means that can be used alone or in conjunction with another Access Device to obtain entry.

  - Applicable Systems- BES Cyber Systems identified in the annual assessment required by CIP-002 and all cyber equipment identified in the Elawan Energy Electronic Access Controls Plan Required by CIP-003.

  - Escorted Access- Access granted into a secured area to an individual, that does require accompaniment by an authorized person.

  - Exceptional Situations- A situation that involves or threatens to involve one or more of the following, or similar, conditions that impact the safety of BES reliability.

    - A risk of injury or death;

    - Civil unrest;

    - An imminent or existing hardware, software, or equipment failure;

    - Unplanned or unauthorized destruction or damage to a security control;

    - A security incident requiring emergency assistance;

    - A response by emergency services; the enactment of a mutual assistance agreement;

    - An impediment of large-scale workforce availability.

  - Locking Device- A mechanism designed for keeping a barrier (e.g., door, lid, gate, combination lock) in a secured state operated only by an Access Device.

  - Unescorted Access- Access granted into a secured area to an individual without an accompaniment.

  - Visitor- Personnel without Unescorted Access granted by CIP Senior Manager or designee.

Acronyms

| Acronym | Term |
| --- | --- |
| BES | Bulk Electric System |
| CIP | Critical Infrastructure Protection |
| CIP SI | CIP Sensitive Information |
| NERC | North American Electric Reliability Corporation |

- Procedure

  - Assessment of Need for Physical Security Controls. Elawan Energy is a renewable generation entity. Elawan Energy has implemented physical security controls based on the need to protect facilities from damage and loss. The function of this physical security control plan is to control access and reduce or mitigate the risk of unauthorized individuals entering the site and for the protection of personnel and equipment

  - Identified Needs:

    - Protect entire Facility against loss due to theft.

    - Protect entire Facility against physical vandalism.

    - Protect the generation controls against unauthorized physical access.

    - Protect the Protection System components against unauthorized physical access.

  - Defense In-Depth

    - Elawan Energy has a defense in-depth approach, which consists of the physical security controls at the following layers:

    - Perimeter fence

    - Structures:

      - Operations and Maintenance (O&M) Buildings

      - Substation Control Houses

      - Solar Generation Structures

  - Adequacy of Physical Security Controls

    - Elawan Energy has determined, based on the assessment of need for asset protection, that a defense-in-depth approach to physical security controls is appropriate for protection of Applicable Systems and individual resource’s protection and control systems.

- Physical Security Controls

  - Perimeter Controls

    - The perimeter fence which surrounds the O&M building and substation control house shall serve as a physical security control but not the primary physical security control.

    - The security gate, secured by a Locking Device, which grants access to the O&M building parking lock shall serve as a physical security control but not the primary physical security control.

    - The secured room inside of the O&M building and the substation control house, housing Applicable Systems shall serve as the primary physical security control.

    - Solar Generation structures shall serve as the primary physical security control for the individual resource’s protection and control systems.

  - Entryways

    - Entryways to locations housing Applicable systems and individual resource protection and control systems shall be equipped with a hardened door secured by a Locking Device controlled by an Access Device.

  - Physical Access Control System Condition

    - In the absence of operational personnel with Unescorted Access permissions, entryways to locations housing Applicable Systems and individual resource’s protection and control systems, including perimeter fences, shall be in a closed and locked state.

- Physical Access Management

  - Unescorted Access

    - Individual need for Unescorted Access privileges to locations housing Applicable Systems and individual resource protection and control systems shall be determined by the CIP Senior Manager or designee.

    - Upon granting Unescorted Access the CIP Senior Manager or designee shall:

      - Distribute Access Devices to locations housing Applicable Systems and individual resource protection and control systems as needed and,

      - Document the approval for Unescorted Access via [Authorized Access Log Attachment B]

  - Visitor Access

    - Visitor(s) access to locations housing Applicable Systems and individual resource protection and control systems shall be based on the need determined by Elawan Energy employees assigned to Elawan Energy.

    - Visitor(s) granted Escorted Access shall be Escorted by a designated Elawan Energy employee assigned to Elawan Energy with Unescorted Access (Escort).

    - Escort shall complete the [Visitors Log Attachment A] prior to entering locations housing Applicable Systems.

- Exceptional Situations

  - Exceptional Situations will be handled in accordance with the process contained in the CIP Exceptional Circumstances and Exceptional Situations section of the CIP Master Policy.

- Access Device Management - Elawan Energy utilizes multiple Access Devices and processes for accessing structures housing Applicable Systems and their associated protection and control equipment.

  - The CIP Senior Manager or designee shall:

    - Ensure all non-issued Access Devices are stored in a secured location.

    - Issue Access Devices to personnel approved for Unescorted Access, as needed.

    - Ensure workstations with access management software installed are in a secured location.

    - Update Access Devices used to unlock Locking Devices, as needed.

    - Revoke, remove or change Access Device privileges as appropriate (i.e. changes in job function, lost or damaged Access Devices, compromised proximity card, etc.), and

    - Document issuances/changes to status of Access Devices owed by Elawan Energy. within the [Authorized Access Log Attachment B].

- Document Retention:

  - Documentation related to physical security controls including, Access Device Logs shall be maintained for a period of three calendar years unless otherwise specified.

Attachment A: Visitor Access Log To be completed by personnel with approved Unescorted Access.

| VISITOR ACCESS LOG |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Site Manager: |  |  |  | Phone: |  |  |
| Visitor Name | Company | Escort | Date | Time In | Time Out | Phone Number |

Attachment B: Authorized Access Log

Log shall be completed once access credentials are issued to an individual that has been granted Unescorted Access.

| AUTHORIZED ACCESS LOG |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Site Manager: |  |  |  | Phone: |  |  |  |
| Access Granted To | Date Granted | Granted By | Access Device Issued Y/N | Issued By | Date Access Removed | Access Devices Returned Y/N | Comments |

Document Control:

| Name | Title | Date |
| --- | --- | --- |
| Leon Wagner | O&M US Operation Director | 04/01/2024 |

| Version | Approval Date | Effective Date | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 04/10/2022 | 04/15/2022 | Initial Physical Security Plan |
| 2.0 | 04/01/2024 | 04/02/2024 | Reviewed Physical Security Plan |