TO:		Kourtnee Jinks, Attorney

Legal Division

FROM:	Patricia Garcia, Infrastructure Analysis Section Director

Infrastructure Division

DATE:	August 15, 2022

RE:	Docket No. 53594 – Petition of Montgomery County Land Investments, Inc. to Amend T&W Water Service Company’s Certificate of Convenience and Necessity in Montgomery County by Expedited Release

On May 11, 2022, Montgomery County Land Investments Inc. (MCLI) filed a petition for streamlined expedited release from T&W Water Service Company's (T&W) water Certificate of Convenience and Necessity (CCN) No. 12892 in Montgomery County, under Texas Water Code (TWC) § 13.2541(b) and 16 Texas Administrative Code (TAC) § 24.245(h). MCLI asserts that the land is at least 25 contiguous acres, is not receiving water service, and is located in Montgomery County, which is a qualifying county.

Based on the mapping review and my technical and managerial review of the information provided by MCLI, I recommend the petition be deemed insufficient for filing and found administratively incomplete.

Mapping Content:

Based on the mapping review by Dave Babicki, Infrastructure Division, the maps and the digital mapping data submitted with Item 5 on July 12, 2022 are deficient. Mr. Babicki determined an acreage difference of approximately 2.8 acres between the acreage conveyed in the deed and the acreage calculated in the digital mapping data. Staff confirmed that the acreage difference calculated by the petitioner is the result of their use of parcel data from Montgomery County Appraisal District to generate the shapefile. To accurately reflect the acreage conveyed in the deed and properly release a tract of land, the digital mapping data requested below must be generated from a metes and bounds survey certified by a licensed state land surveyor or registered professional land surveyor.

MCLI must submit the following items to resolve the mapping deficiencies:

- A general location map identifying only the tract of land, in reference to the nearest county boundary, city, or town.

- A detailed map identifying only the tract of land, in reference to verifiable man-made and natural landmarks, such as roads, rivers, and railroads.

- Digital mapping data for the tract of land, as a single polygon record, in shapefile (SHP) format, georeferenced in either NAD83 Texas Statewide Mapping System (Meters) or NAD83 Texas State Plane Coordinate System (US Feet).

Staff recommends MCLI obtain additional mapping guidance from the PUC’s mapping staff, Dave Babicki by email at dave.babicki@puc.texas.gov to resolve the mapping deficiencies.

Staff will need at least 30 days to review the documentation, maps, and digital data provided by MCLI and draft a recommendation.