Palo Duro Service Company, Inc.

3505 Williams Road

Fort Worth, Texas 76116

(817) 244-2248

Public Utility Commission of Texas

P.O. Box 13326

Austin, TX 78711-3326

Texas Division of Emergency Management

1033 La Posada Suite 300

Austin, TX 78752

Parker County Office of Emergency Management

215 Trinity Avenue

Weatherford, TX 76086

Wise County Office of Emergency Management

205 North State Street

Decatur, TX 76234

Tri-County Electric Cooperative

200 Bailey Ranch Road

Aledo, Texas 76008

TXU Energy

6555 Sierra Drive

Irving, TX, 75039

Regarding: PUCT Project 53938 Critical Facilities Information

To Whom It May Concern:

In compliance with 2022 PUCT Project 53938, Critical Load Facilities Information for Palo Duro Service Company, Inc. (CCN 12200) and its associated Public Water Systems identified below is being provided to the entities addressed above.

Aledo Ridge (PWS #1840185)

Aledo Ridge, located in Parker County, is a public water system with a connection count of 26. Accessed from White Settlement Road, the water system shows to be serviced by two water wells, G1840185A which produces 18 gallons per minute, and G1840185B which produces 17 gallons per minute for a combined production of 35 gallons per minute. Facilities consists of one 900 gallon and one 220-gallon hydro-pneumatic tank, and three distribution pumps all rated at 40 gallons per minute. Two 20,000 GSTs provide storage.

Glider Base Estates (PWS #2490028)

Glider Base Estates, located in Wise County, is a public water system with a connection count of 129. The water system is serviced by four water wells, G2490028A produces 9 gallons per minute, G2490028B produces 10 gallons per minute, G2490028C produces 8 gallons per minute, and G2490028D produces 9 gallons per minute, for a combined production of 36 gallons per minute. Facilities consist of two plants, one at Loren Ryan Drive with a 25,000-gallon GST and pump facilities and one at Circle Del Rio which has one pump facility.

Duchane Chateaux (PWS #1840100)

Duchane, located in Parker County, is a public water system with a connection count of 101. Accessed from Brandon Drive, the water system is be serviced by four water wells, G1840100B produces 4 gallons per minute, G1840100C produces 10 gallons per minute, G1840100D produces 15 gallons per minute, and G1840100E produces 17 gallons per minute, for a combined production of 46 gallons per minute. Facilities consists of 2 plants.  Brandon Drive has two 20,000-gallon GSTs and pump facilities and Owen Circle has one 10,000-gallon GST with pump facilities.

Trinity River Estates (PWS #1840099)

Trinity River Estates, located in Parker County, is a public water system with a connection count of 30. Accessed from Kelly Ct, the water system is serviced by two water wells, G1840099A which produces 12 gallons per minute and G1840099B which produces 14 gallons per minute, for a combined production of 26 gallons per minute. Facilities consist of two 220-gallon hydro-pneumatic tanks, and two distribution pumps rated at 30 gallons per minute and 40 gallons per minute. A small 2500-gallon GST provides storage.

Contact Information:

Primary Contact:

Shirly Rodriguez, Office Manager

Office Phone: (817) 244-2248

Cell Phone: (239) 265-0809

Alternate Contact:

William Lena, Field Operations Manager

Cell Phone: (817) 901-3814

Mailing Address:

3505 Williams Road

Fort Worth, Texas 76116

If additional information is needed, please contact me at your convenience.

Thank you.

Rhett Micheletti, President

Palo Duro Service Company, Inc.

Cell Phone: (832) 541-5909