Office

Memorandum

| Date: | May 30, 2023 |  |  |
| --- | --- | --- | --- |
| To: | File |  |  |
| From: | Brenda J. Perkins |  |  |
| Subject: | Alternative Routes Evaluation: Ramhorn Hill-Dunham 345 kV Transmission Line Project |  |  |

_______

This memorandum discusses my evaluation of routing alternatives for Oncor Electric Delivery Company LLC’s (“Oncor’s”) proposed Ramhorn Hill-Dunham 345 kV Transmission Line Project (“Proposed Transmission Line Project”).  In addition to the recommendation for a route that best meets the requirements of the Texas Utilities Code and the Substantive Rules of the Public Utility Commission of Texas (“Commission”), I also selected alternative routes to be filed with this CCN Application.

The goal of this process is to provide the Commission with an adequate number of alternative routes to conduct a proper evaluation.  These alternative routes provide good geographic diversity while complying with Section 37.056(c)(4)(A)-(D) of the Texas Utilities Code, Commission Procedural Rule 22.52(a)(4), and Commission Substantive Rule 25.101(b)(3)(B), including the Commission’s policy of prudent avoidance.

My recommendations are based on my reconnaissance and observations of the project area, my independent review of the data included in the Environmental Assessment and Alternative Route Analysis for Oncor Electric Delivery Company LLC’s Proposed Ramhorn Hill Switch-Dunham Switch 345 kV Transmission Line Project in Denton and Wise Counties, Texas (“Environmental Assessment and Routing Study”), prepared by Halff Associates, Inc. (“Halff”), my discussions with Halff personnel, my discussions with Oncor personnel, my participation in the public participation meeting process, my review of correspondence related to the Proposed Transmission Line Project, my understanding of other input that Oncor received from interested parties, and other information.  My recommendation incorporates consideration of engineering feasibility, the estimated cost of alternative routes, construction limitations, and other information.

Halff documented its efforts to identify potential preliminary alternative routes for the Proposed Transmission Line Project in Section 4.0 of the Environmental Assessment and Routing Study.  After Halff completed the initial data gathering and constraints mapping process, they identified preliminary alternative route links on recent aerial photography obtained from NearMap (available through Halff’s subscription service).  These preliminary alternative route links were selected considering the location of existing corridors, apparent property boundaries and routing constraints.  Some of the routing constraints within the study area are:  United States Army Corp of Engineers (“USACE”) owned recreational and environmentally sensitive land; many major highways where 90-degree roadway crossings by transmission lines are required by the Texas Department of Transportation; oil and gas facilities; existing and developing residential and commercial areas; aircraft landing facilities; as well as other constraints.  Numerous preliminary alternative route links were identified by Halff, prior to the public participation meetings, that when combined, formed many preliminary alternative routes to connect the proposed Ramhorn Hill Switch to the proposed Dunham Switch.  The preliminary alternative route links evaluated by Halff and presented at the public participation meetings are depicted in Figures 6-1 through 6-8 located in Appendix C of the Environmental Assessment and Routing Study, along with the alternative route link deletions, additions and modifications that were made following the public participation meetings.  The modified preliminary alternative route links are discussed in detail in Section 6.0 of the Environmental Assessment and Routing Study and are briefly summarized below.

In general, links were modified where possible to address public comments and routing constraints identified after additional field investigations.  Following the preliminary alternative route link revisions, a total of 140 alternative route links were adopted.  Halff identified several hundred thousand alternative routes using these route links.  Through an iterative process that considered route length, constraints data, input from public meetings, and information from local, state, and federal officials, Halff and Oncor reduced the total number of route combinations to a smaller subset of geographically diverse and forward progressing alternative routes that were further evaluated, as discussed in Section 7.0 of the Environmental Assessment and Routing Study.  A total of 221 alternative routes were selected for further analysis as provided in Table 7-2 in Appendix E of the Environmental Assessment and Routing Study.

Each of the 221 preliminary alternative routes identified possesses both positive and negative comparative attributes.  I considered these attributes to select a set of geographically diverse routing alternatives to be filed as a part of this Application.  Each alternative route complies with Section 37.056(c)(4)(A)-(D) of the Texas Utilities Code and the Commission’s Substantive Rule 25.101, including the Commission’s policy of prudent avoidance.

Below, I discuss the alternative routes that I selected to be filed with the Application.  The alternative routes can be grouped in many different ways; one approach is the grouping of alternative routes into geographic corridors.  I grouped the alternative routes into six different geographic corridors using State Highway (“SH”) 114 as the division between north and south corridor designation.  These six corridors are identified as the: (1) north corridor using Link M8; (2) north corridor using Link M7; (3) north corridor using Link M6; (4) south corridor using Link M5; (5) south corridor using Link M4; and (6) south corridor using Link M3 (see map attached to this Memorandum for these alternative route link locations).  All alternative routes cross several major highways due to the location of this project’s endpoints being on opposite sides of these highways:  United States Highway (“US”) 377, Interstate Highway (“IH”) 35W, Farm to Market (“FM”) 156, SH 114, and US 287/81.

I selected 74 geographically diverse alternative routes to be filed with the CCN Application to allow for an adequate number of alternative routes to conduct a proper evaluation.  The links that comprise these alternative routes are presented in Table 1, attached to this Memorandum.  Table 2, attached to this Memorandum, presents quantifiable environmental data on the 74 alternative routes filed as a part of the CCN Application.  The filed alternative routes use each of the 140 alternative links in at least one route.

I then presented these 74 alternative routes to Oncor’s engineer overseeing this project, Ms. Amy Zapletal, for consideration of engineering feasibility, construction limitations, and alternative route cost estimates.  Below is a discussion of each of the six geographic corridors and the alternative routes selected for filing within each corridor.

The north corridor routes containing Link M8 (“Link M8 Corridor Routes”) vary in length from approximately 20.8 to 22.5 miles.  Transmission line costs for Link M8 Corridor Routes range from $166,165,000 to $178,245,000.  Link M8 Corridor Routes contain the greatest number of habitable structures within 500 feet of the route centerline with numbers varying from 188 to 400.  The 11 alternatives filed in the Application from the Link M8 Corridor Routes include Alternative Routes 1, 65, 67, 68, 69, 72, 96, 142, 143, 191 and 192.

The north corridor routes containing Link M7 (“Link M7 Corridor Routes”) vary in length from approximately 20.5 to 22.5 miles. Transmission line costs for Link M7 Corridor Routes range from $167,945,000 to $188,738,000.  Link M7 Corridor Routes vary in the number of habitable structures within 500 feet of the route centerline from 108 to 327.  The 17 alternatives filed in the Application from the Link M7 Corridor Routes include Alternative Routes 41, 42, 54, 71, 86, 94, 103, 138, 175, 176, 184, 185, 207, 217, 218, 219 and 221.

The north corridor routes using Link M6 (“Link M6 Corridor Routes”) include the longest filed route (Route 216) with routes within this corridor varying in length from approximately 20.4 to 22.9 miles.  Transmission line costs for Link M6 Corridor Routes range from $171,340,000 to $237,423,000.  Link M6 Corridor Routes vary in the number of habitable structures within 500 feet of the route centerline from 145 to 364.  The 12 alternatives filed in the Application from the Link M6 Corridor Routes include Alternative Routes 10, 11, 19, 33, 92, 117, 154, 170, 178, 186, 187 and 216.

The south corridor routes containing Link M5 (“Link M5 Corridor Routes”) contain the shortest filed route (Route 16) with route lengths varying from approximately 19.9 to 22.5 miles. Transmission line costs for Link M5 Corridor Routes range from $172,350,000 to $208,670,000.  Link M5 Corridor Routes vary in the number of habitable structures within 500 feet of the route centerline from 132 to 271.  The 9 alternatives filed in the Application from the Link M5 Corridor Routes include Alternative Routes 5, 16, 25, 26, 28, 61, 108, 146, and 200.

The south corridor routes using Link M4 (“Link M4 Corridor Routes”) vary in length from approximately 20.0 to 22.2 miles. Transmission line costs for Link M4 Corridor Routes range from $172,226,000 to $212,686,000.  Link M4 Corridor Routes vary in the number of habitable structures within 500 feet of the route centerline from 151 to 266.  The 10 alternatives filed in the Application from the Link M4 Corridor Routes include Alternative Routes 3, 15, 24, 36, 44, 58, 63, 70, 78 and 137.

The south corridor routes using Link M3 (“Link M3 Corridor Routes”) vary in length from approximately 20.6 to 22.5 miles. Transmission line costs for Link M3 Corridor Routes range from $164,581,000 to $224,991,000.  Link M3 Corridor Routes contain the least number of habitable structures within 500 feet of the route centerline with numbers varying from 93 to 205.  The 15 alternatives filed in the Application from the Link M3 Corridor Routes include Alternative Routes 13, 14, 18, 22, 23, 29, 43, 87, 116, 119, 130, 132, 164, 179 and 199.

After analyzing each of the 74 routes within the six geographic corridors, I selected Route 179 of the Link M3 Corridor Routes as the route that best meets the requirements of Texas Utilities Code Section 37.056 (c)(4)(A)-(D) and the Commission Substantive Rule 25.101(b)(3)(B).  Route 179 is comprised of Links A0-A4-B1-B61-B62-C1-C21-C23-C7-E2-E1-E6-G1-G3-H41-H42-H8-I8-J3-K1-L5-L4-L3-L2-M1-M2-M3-R4-V2-Z.

Some of the significant factors which led to the selection of Route 179 include the following:

- The length of Route 179 is approximately 21.8 miles, which is only 1.9 miles longer than the shortest among all the filed routes (Route 16) and approximately 1.1 miles shorter than the longest alternative route included in the Application (Route 216 is the longest at approximately 22.9 miles);

- The transmission line estimated cost for Route 179 is $175,208,000, which is approximately 6.5% more than the least expensive alternative route (Route 29 estimated at $164,581,000) and is approximately 35.5% less than the most expensive alternative route (Route 117 estimated at $237,423,000);

- There are 97 habitable structures within 500 feet of the centerline of Route 179, which is only four more than the route with the least number (95 for Route 164) and 303 less than the route with the highest number (400 for Route 192);

- Route 179 parallels existing compatible corridors for 23% of its length (including apparent property boundaries).  Route 117 possesses the highest percentage parallel to existing corridors (40%), but is longer in route length (22.7 miles) and has a higher number of habitable structures within 500 feet its centerline (263).  Route 221 had the lowest percentage (17%) parallel to existing corridors;

- Route 179 has 4,607 feet of its route through commercial/industrial areas.  Route lengths through commercial/industrial areas varied from 4,085 feet (Route 219) to 14,702 feet (Route 117);

- Route 179 crosses 20,248 feet of cropland/hay meadow and crosses 71,051 feet of rangeland pasture.  Route lengths crossing cropland/hay meadow varied from 12,347 feet (Route 164) to 36,231 feet (Route 69).  Route lengths crossing rangeland pasture varied from 46,458 feet (Route 26) to 76,318 feet (Route 187);

- Route 179 crosses 10,126 feet of upland woodlands and has 7,162 feet of its route through riparian areas.  Route 26 has the greatest length (15,960 feet) of its route across upland woodlands and Route 28 has the greatest length (15,718 feet) of its route across riparian areas.  The Link M5 Corridor Routes contain the greatest length across upland woodlands and riparian areas which are associated with the floodplain of Elizabeth Creek;

- Route 179 has no length of its route across potential wetlands (57 of the filed routes cross potential wetlands, with Routes 92 and 218 having the highest crossing length of 849 feet);

- Route 179 has 27 streams crossed by its centerline (the greatest number of streams crossed within the filed routes is 33);

- The length of Route 179 that is parallel to streams (within 100 feet) is 1,351 feet (the greatest amount of route length parallel to streams within the filed routes is 5,108 feet);

- Route 179 has 1,704 feet of its route across lakes or ponds (open waters). Route 185 has the greatest length (2,080 feet) across lakes or ponds of the filed routes;

- Route 179 has one known rare/unique plant location within the route right-of-way.  Nine of the filed routes have four known rare/unique plant locations within the route right-of-way;

- Route 179 has one recorded cultural resource site crossed by its centerline (34 of the filed routes have one recorded cultural resource site crossed by their centerline);

- Route 179 has three recorded cultural resource sites within 1,000 feet of its centerline (all filed routes have at least one recorded cultural resource site within 1,000 feet of their centerline and two of the filed routes have five recorded cultural resource sites within 1,000 feet of their centerline);

- Route 179 has three FAA-registered airports with a runway greater than 3,200 feet within 20,000 feet of the centerline along its entire length (all filed routes have at least three FAA-registered airports with a runway greater than 3,200 feet within 20,00 feet of their centerline, with some filed routes having four);

- Route 179 has four FAA-registered airports with no runway greater than 3,200 feet within 10,000 feet of the centerline along its entire length (three of the filed routes have six FAA-registered airports with no runway greater than 3,200 feet within 10,000 feet of their centerline);

- Route 179 has two heliports located within 5,000 feet of its centerline (the range of heliports within 5,000 feet of the filed route centerlines is one to three);

- Route 179 has two electronic installations within 2,000 feet of its centerline (the range in electronic installations within 2,000 feet of the filed route centerlines is 0 to 6);

- Route 179 crosses nineteen U.S. or State Highways along its entire length (the greatest number of U.S. or State Highways crossings is twenty);

- Route 179 crosses eleven FM, county roads or other streets along its entire length (the greatest number of FM, county roads or other street crossings is fourteen); and

- Route 179 has been judged to be feasible from an engineering perspective based on currently known conditions, without the benefit of on-the-ground and subsurface surveys, and there are no currently identifiable engineering constraints that impact this alternative route that cannot be addressed with additional consideration by Oncor during the engineering and construction process.

Additional information concerning the issues addressed in this memorandum can be found in the Environmental Assessment and Routing Study, included as Attachment No. 1 to the CCN Application.

After considering all of the parameters and issues as discussed in this memo, I selected Route 179 as the alternative route that best meets the requirements of the Texas Utilities Code and the Commission’s Substantive Rules.